# Experiment Set 3 — Step-by-Step Lab Guide (Arduino UNO)

Submission due: 17/10/2026. This set adds display output (7-segment LED, 16x2 LCD), serial communication in both directions (Serial Monitor / Plotter / typed input), distance sensing (HC-SR04 ultrasonic) and internet data (ThingSpeak upload, and one Arduino reading another Arduino's web page). Wiring is given as direct component-to-component connections — build each connection in order, landing both ends of a "connection" in the same breadboard row (or a power rail) to join them electrically.

This guide tells you how to build and run each experiment. The graded report questions from the PDF are listed at the end of each experiment as a checklist — answer those yourself (they need your own explanation, photos/video, and measurements). Programs for Experiments 21, 26 (partly), 27, 28, 29, 30 and 31 are not printed in the slide deck. The deck prints the code only for Experiments 22, 23, 24 and the Serial Read example used by 26. For the others, the slides give just the task and the questions, and refer to sketches named Experiment_xx supplied separately by the instructor. The code shown below for those is a standard, working Arduino implementation that matches the described behaviour and the PDF's own questions — swap in the instructor's exact file if it differs; the wiring and concepts stay the same.

# 1. Components Used

Same physical kit as Sets 1 and 2 (component-images/), plus these items that are now active:

| Component | Photo | Pins / Legs |
| --- | --- | --- |
| Arduino UNO | component-images/arduino-uno.jpeg | Left header: 5V, RESET, 3.3V, 5V, GND, GND, VIN. Bottom header: A0–A5. Right headers: SCL, SDA, AREF, GND, 13, 12, 11~, 10~, 9~, 8 and 7, 6~, 5~, 4, 3~, 2, TX→1, RX←0. ~ = PWM-capable pin. |
| Breadboard | component-images/breadboard.jpeg | Top/bottom rails: red `+`, blue `−`. Center strips: rows A–E / F–J, columns 1–60; A–E not joined to F–J. |
| LED, resistor (220 Ω), jumper wires, USB cable | led-light.jpeg, resistor.jpeg, wire.jpeg, arduino-connector.jpeg | LED: long leg = anode (+), short leg = cathode (−). Resistors have no polarity. |
| Potentiometer (10 kΩ) | component-images/potentiometer.jpeg | 3 legs: two outer legs = fixed ends of the track (across 5V/GND), middle leg = wiper. |
| Push button switch | component-images/switch.jpeg | 4 legs: the 2 legs on the same side of the body are always joined; the two sides join only while the button is pressed. |
| 7-segment LED display (1 digit) | component-images/seven-segment-front.jpeg, seven-segment-back.jpeg | 10 pins — 5 on the top edge, 5 on the bottom edge. Holding the display with the digit facing you and the decimal point at bottom-right, the pin numbers run 1–5 along the bottom (left → right) and 6–10 along the top (right → left). Standard pin-out: 1 = E, 2 = D, 3 = COM, 4 = C, 5 = DP, 6 = B, 7 = A, 8 = COM, 9 = F, 10 = G. The two COM pins (3 and 8) are joined inside — use either one. |
| HC-SR04 ultrasonic sensor | component-images/ultrasonic-sensor.jpeg | 4 pins: Vcc, Trig, Echo, Gnd. Vcc = +5 V; Trig is an input (hold HIGH 10 µs to fire a burst); Echo is an output (goes HIGH for as long as the echo takes to return). Works at 40 kHz. |
| 16x2 LCD (JHD 162A, HD44780-compatible) | component-images/led-screen-front.jpeg, led-back.jpeg | 16 pins, numbered 1–16 on the back: 1 VSS, 2 VDD (VCC), 3 V0 (VEE, contrast), 4 RS, 5 R/W, 6 E, 7–14 DB0–DB7, 15 LED+ (backlight anode), 16 LED− (backlight cathode). |
| Ethernet Shield (W5100) | (not in image set) | Stacks on the UNO. Uses pins 10, 11, 12, 13 (SPI) internally — those four are reserved while the shield is mounted. RJ45 port connects to your router. Needed for Experiments 29 and 31. |
| Second Arduino + Ethernet Shield | (not in image set) | Needed only for Experiment 31 — acts as the server the first board reads from. |

# 2. One-Time Software Setup

| Step | From | To |
| --- | --- | --- |
| 1 | Arduino UNO USB port | USB cable (USB-B end) |
| 2 | USB cable (USB-A end) | PC USB port |
| 3 | IDE menu Tools → Board | Select Arduino AVR Boards → Arduino Uno |
| 4 | IDE menu Tools → Port | Select COM3 (Arduino UNO) (use whichever COM number appears) |
| 5 (Exp 29, 31) | Ethernet Shield | Push firmly onto the UNO headers, then connect its RJ45 port to your router |
| 6 (Exp 29) | Browser | Use the ThingSpeak account and channel created in Experiment 20 (Field 1 = distance) |

Serial Monitor / Plotter (Experiments 22–29): open from Tools → Serial Monitor (Ctrl+Shift+M) or Tools → Serial Plotter (Ctrl+Shift+L), and set the baud rate to 9600 at the bottom right. The two windows cannot be open at the same time — close one before opening the other.

# 3. Breadboard Layout Key

Every "connection" below means: plug both ends into the breadboard so they land in the

same row (or the same power rail).

Power rails: red strip = + (tied to Arduino 5V), blue strip = − (tied to Arduino GND).

Always keep a resistor in series with every LED or LED segment (220 Ω) — an Arduino pin can

safely source only about 20 mA.

# 4. Experiment-by-Experiment Build

## Experiment 21 — One 7-Segment Display Counting 0 to 9 (every 2 s)

Each of the 7 segments (a–g) gets its own Arduino pin and its own 220 Ω resistor; the display's common pin goes to GND. (This wiring is for a common-cathode display — see Q21.b to check which kind you have; a common-anode display needs the change in the note below.)

| # | Connection |
| --- | --- |
| 1 | Power the Arduino UNO from the PC using the USB cable |
| 2 | Arduino Digital Pin 2 — one end of Resistor 1 |
| 3 | Resistor 1 other end — display pin 7 (segment a) |
| 4 | Arduino Digital Pin 3 — one end of Resistor 2 |
| 5 | Resistor 2 other end — display pin 6 (segment b) |
| 6 | Arduino Digital Pin 4 — one end of Resistor 3 |
| 7 | Resistor 3 other end — display pin 4 (segment c) |
| 8 | Arduino Digital Pin 5 — one end of Resistor 4 |
| 9 | Resistor 4 other end — display pin 2 (segment d) |
| 10 | Arduino Digital Pin 6 — one end of Resistor 5 |
| 11 | Resistor 5 other end — display pin 1 (segment e) |
| 12 | Arduino Digital Pin 7 — one end of Resistor 6 |
| 13 | Resistor 6 other end — display pin 9 (segment f) |
| 14 | Arduino Digital Pin 8 — one end of Resistor 7 |
| 15 | Resistor 7 other end — display pin 10 (segment g) |
| 16 | Display pin 3 or 8 (common) — Arduino GND |

Segment layout (for reference): a = top bar, b = top-right, c = bottom-right, d = bottom bar, e = bottom-left, f = top-left, g = middle bar, DP = decimal point (left unused).

Code — sketch name `Experiment_21`:

// segments a,b,c,d,e,f,g on Arduino pins 2..8
const int segPins[7] = {2, 3, 4, 5, 6, 7, 8};
 
// bit 0 = a, bit 1 = b, ... bit 6 = g  (1 = segment ON on a common-cathode display)
const byte digits[10] = {
  0x3F,  // 0
  0x06,  // 1
  0x5B,  // 2
  0x4F,  // 3
  0x66,  // 4
  0x6D,  // 5
  0x7D,  // 6
  0x07,  // 7
  0x7F,  // 8
  0x6F   // 9
};
 
void setup() {
  for (int i = 0; i < 7; i++) {
    pinMode(segPins[i], OUTPUT);
  }
}
 
void loop() {
  for (int n = 0; n < 10; n++) {
    for (int s = 0; s < 7; s++) {
      digitalWrite(segPins[s], bitRead(digits[n], s));
    }
    delay(2000);   // change the number every two seconds
  }
}

Common-anode display? Wire the common pin to 5V instead of GND and invert the output: change the write to digitalWrite(segPins[s], !bitRead(digits[n], s));. Everything else is identical.

Report checklist: 21.a (video with voice-over showing the display counting 0→9 and repeating), 21.b (how to tell Common Cathode from Common Anode — e.g. with a multimeter in diode-test mode, touch the red probe to the common pin and the black probe to a segment pin: if that segment lights, the display is common anode; if it only lights with the probes swapped (black on the common pin, red on a segment), it is common cathode. Alternatively, without a multimeter: join Arduino 5V → 220 Ω → one segment pin and touch the common pin to GND; the segment lighting means common cathode. If it stays dark, reverse it (common pin to 5V, segment pin through 220 Ω to GND) — lighting then means common anode).

## Experiment 22 — Potentiometer Value on Serial Monitor and Serial Plotter

| # | Connection |
| --- | --- |
| 1 | Power the Arduino UNO from the PC using the USB cable |
| 2 | Potentiometer outer leg A — Arduino 5V |
| 3 | Potentiometer outer leg B — Arduino GND |
| 4 | Potentiometer wiper (middle leg) — Arduino A0 |

Code — sketch name `Experiment_22` (from the slide deck, verbatim):

/*
AnalogReadSerial
Reads an analog input on pin 0, prints the result to the Serial Monitor.
Graphical representation is available using Serial Plotter (Tools > Serial Plotter menu).
Attach the center pin of a potentiometer to pin A0, and the outside pins to +5V and ground.
This example code is in the public domain.
http://www.arduino.cc/en/Tutorial/AnalogReadSerial
*/
// the setup routine runs once when you press reset:
void setup() {
  // initialize serial communication at 9600 bits per second:
  Serial.begin(9600);
}
 
// the loop routine runs over and over again forever:
void loop() {
  // read the input on analog pin 0:
  int sensorValue = analogRead(A0);
  // print out the value you read:
  Serial.println(sensorValue);
  delay(1);        // delay in between reads for stability
}

How to observe it: upload, open Tools → Serial Monitor (9600 baud) and turn the knob — the numbers run from 0 to 1023. Close it, open Tools → Serial Plotter and the same values are drawn as a live line graph.

Report checklist: 22.a (video with voice-over), 22.b (22.b.1 Serial.begin(9600); opens the USB serial port at 9600 bits per second; 22.b.2 Serial.println(..) sends the value as readable text followed by a carriage-return + newline).

## Experiment 23 — Push Button State on Serial Monitor and Plotter

| # | Connection |
| --- | --- |
| 1 | Power the Arduino UNO from the PC using the USB cable |
| 2 | Resistor (10 kΩ recommended, 220 Ω also works) — one end to Arduino 5V |
| 3 | Resistor other end — switch Side A (its two same-side legs, internally joined) (this row is the junction) |
| 4 | Switch Side B (its two same-side legs, internally joined) — Arduino GND |
| 5 | Junction of resistor and switch (step 3) — Arduino Digital Pin 2 |

The resistor pulls pin 2 up to 5V while the button is open, and pressing the button connects that junction straight to GND. So the input reads 1 when released and 0 while pressed (the opposite of Experiment 9's wiring).

Code — sketch name `Experiment_23` (from the slide deck, verbatim):

/*
DigitalReadSerial
Reads a digital input on pin 2, prints the result to the Serial Monitor
This example code is in the public domain.
http://www.arduino.cc/en/Tutorial/DigitalReadSerial
*/
// digital pin 2 has a pushbutton attached to it. Give it a name:
int pushButton = 2;
 
// the setup routine runs once when you press reset:
void setup() {
  // initialize serial communication at 9600 bits per second:
  Serial.begin(9600);
  // make the pushbutton's pin an input:
  pinMode(pushButton, INPUT);
}
 
// the loop routine runs over and over again forever:
void loop() {
  // read the input pin:
  int buttonState = digitalRead(pushButton);
  // print out the state of the button:
  Serial.println(buttonState);
  delay(1);        // delay in between reads for stability
}

How to observe it: Serial Monitor scrolls 1 continuously, and 0 while you hold the button. In the Serial Plotter you see a flat line at 1 that drops to 0 for as long as the button is pressed.

Report checklist: 23.a (video with voice-over — show both the Monitor and the Plotter reacting to the button).

## Experiment 24 — Potentiometer → Serial Monitor + LED Brightness (with `map`)

| # | Connection |
| --- | --- |
| 1 | Power the Arduino UNO from the PC using the USB cable |
| 2 | LED anode (long leg) — one end of the resistor |
| 3 | Resistor other end — Arduino Digital Pin 9 (`9~`, PWM) |
| 4 | LED cathode (short leg) — Arduino GND |
| 5 | Potentiometer outer leg A — Arduino 5V |
| 6 | Potentiometer outer leg B — Arduino GND |
| 7 | Potentiometer wiper (middle leg) — Arduino A0 |

Code — sketch name `Experiment_24` (from the slide deck, verbatim):

/*
Analog input, analog output, serial output
Reads an analog input pin, maps the result to a range from 0 to 255 and uses
the result to set the pulse width modulation (PWM) of an output pin.
Also prints the results to the Serial Monitor.
The circuit:
- potentiometer connected to analog pin 0.
Center pin of the potentiometer goes to the analog pin.
side pins of the potentiometer go to +5V and ground
- LED connected from digital pin 9 to ground
created 29 Dec. 2008
modified 9 Apr 2012
by Tom Igoe
This example code is in the public domain.
http://www.arduino.cc/en/Tutorial/AnalogInOutSerial
*/
// These constants won't change. They're used to give names to the pins used:
const int analogInPin = A0;  // Analog input pin that the potentiometer is attached to
const int analogOutPin = 9;  // Analog output pin that the LED is attached to
 
int sensorValue = 0;         // value read from the pot
int outputValue = 0;         // value output to the PWM (analog out)
 
void setup() {
  // initialize serial communications at 9600 bps:
  Serial.begin(9600);
}
 
void loop() {
  // read the analog in value:
  sensorValue = analogRead(analogInPin);
  // map it to the range of the analog out:
  outputValue = map(sensorValue, 0, 1023, 0, 255);
  // change the analog out value:
  analogWrite(analogOutPin, outputValue);
 
  // print the results to the Serial Monitor:
  Serial.print("sensor = ");
  Serial.print(sensorValue);
  Serial.print("\t output = ");
  Serial.println(outputValue);
 
  // wait 2 milliseconds before the next loop for the analog-to-digital
  // converter to settle after the last reading:
  delay(2);
}

Report checklist: 24.a (video with voice-over — show the LED dimming/brightening and the Monitor/Plotter values), 24.b (24.b.1 map(sensorValue, 0, 1023, 0, 255) linearly rescales the 10-bit ADC value (0–1023) to the 8-bit PWM range (0–255); 24.b.2 Serial.print("\t output = "); prints the text, where \t is a tab character, with no line break; 24.b.3 Serial.println(outputValue); prints the number and then ends the line).

## Experiment 25 — Same as 24, but Without `map()`

Hardware is identical to Experiment 24 (LED + resistor on pin 9, potentiometer wiper on A0, outer legs on 5V/GND).

| # | Connection |
| --- | --- |
| 1 | Power the Arduino UNO from the PC using the USB cable |
| 2 | LED anode (long leg) — one end of the resistor |
| 3 | Resistor other end — Arduino Digital Pin 9 (PWM) |
| 4 | LED cathode (short leg) — Arduino GND |
| 5 | Potentiometer outer leg A — Arduino 5V |
| 6 | Potentiometer outer leg B — Arduino GND |
| 7 | Potentiometer wiper (middle leg) — Arduino A0 |

Code (Experiment 24 with the map() line replaced by a plain division — 1023 ÷ 4 ≈ 255, which does the same scaling):

const int analogInPin = A0;
const int analogOutPin = 9;
 
int sensorValue = 0;
int outputValue = 0;
 
void setup() {
  Serial.begin(9600);
}
 
void loop() {
  sensorValue = analogRead(analogInPin);
  outputValue = sensorValue / 4;          // 0..1023  ->  0..255, no map()
  analogWrite(analogOutPin, outputValue);
 
  Serial.print("sensor = ");
  Serial.print(sensorValue);
  Serial.print("\t output = ");
  Serial.println(outputValue);
 
  delay(2);
}

(Integer division makes outputValue run 0…255. analogWrite(pin, sensorValue / 4) directly, or sensorValue >> 2, is equivalent.)

Report checklist: 25.a (video with voice-over).

## Experiment 26 — Serial Read (Typing to the Arduino)

No extra hardware — only the Arduino on USB. (An LED on pin 13 / the on-board "L" LED can be added if you want a visible response, but it is not required.)

| # | Connection |
| --- | --- |
| 1 | Power the Arduino UNO from the PC using the USB cable |

Code — sketch name `Experiment_26` (the Serial Read example shown on the slides):

int incomingByte = 0;   // for incoming serial data
 
void setup() {
  Serial.begin(9600);     // opens serial port, sets data rate to 9600 bps
}
 
void loop() {
  // send data only when you receive data:
  if (Serial.available() > 0) {
    // read the incoming byte:
    incomingByte = Serial.read();
 
    // say what you got:
    Serial.print("I received: ");
    Serial.println(incomingByte, DEC);
  }
}

How to observe it: open the Serial Monitor at 9600 baud, type a character (for example A) in the input box at the top and press Send. The Arduino replies I received: 65 (the ASCII code of A). With the line-ending box set to Newline, it also prints I received: 10 because the newline character \n is itself a received byte.

Report checklist: 26.a (video with voice-over), 26.b (26.b.1 Serial.available() returns how many bytes are waiting in the serial receive buffer (0 if none); 26.b.2 Serial.read() removes and returns the first waiting byte as an int, or −1 if nothing is available).

## Experiment 27 — Typed Number 3, 4 or 5 Lights the LED on That Pin for 2 Seconds

Three LEDs, each with its own resistor, on pins 3, 4 and 5.

| # | Connection |
| --- | --- |
| 1 | Power the Arduino UNO from the PC using the USB cable |
| 2 | LED 1 anode (long leg) — Resistor 1 one end |
| 3 | Resistor 1 other end — Arduino Digital Pin 3 |
| 4 | LED 1 cathode (short leg) — Arduino GND |
| 5 | LED 2 anode (long leg) — Resistor 2 one end |
| 6 | Resistor 2 other end — Arduino Digital Pin 4 |
| 7 | LED 2 cathode (short leg) — Arduino GND |
| 8 | LED 3 anode (long leg) — Resistor 3 one end |
| 9 | Resistor 3 other end — Arduino Digital Pin 5 |
| 10 | LED 3 cathode (short leg) — Arduino GND |

Code:

void setup() {
  Serial.begin(9600);
  pinMode(3, OUTPUT);
  pinMode(4, OUTPUT);
  pinMode(5, OUTPUT);
  Serial.println("Enter a number between 3 and 5:");
}
 
void loop() {
  if (Serial.available() > 0) {
    String text = Serial.readStringUntil('\n');   // read the whole typed line
    text.trim();                                  // drop spaces / '\r'
    if (text.length() == 0) return;               // ignore an empty line
 
    int number = text.toInt();                    // text -> integer (0 if not a number)
 
    if (number >= 3 && number <= 5) {
      digitalWrite(number, HIGH);                 // pin number == typed number
      Serial.print("LED on pin ");
      Serial.print(number);
      Serial.println(" is ON for 2 seconds.");
      delay(2000);
      digitalWrite(number, LOW);
    } else {
      Serial.println("Number is out of Range.");
    }
  }
}

How to observe it: Serial Monitor at 9600 baud, line ending Newline. Type 3 → LED on pin 3 for 2 s; 4 → pin 4; 5 → pin 5; anything else (2, 9, -1, abc) → Number is out of Range.

Report checklist: 27.a (video with voice-over), 27.b (paste the full new sketch), 27.c (describe the changes vs. Experiment 26: Serial.read() byte-by-byte replaced with reading a whole line and converting it with toInt(); added three output pins; added a range check 3–5; the typed number doubles as the pin number; the out-of-range message; and delay(2000) for the 2-second ON time).

## Experiment 28 — Distance Measurement with the HC-SR04 Ultrasonic Sensor

| # | Connection |
| --- | --- |
| 1 | Power the Arduino UNO from the PC using the USB cable |
| 2 | HC-SR04 Vcc — Arduino 5V |
| 3 | HC-SR04 Gnd — Arduino GND |
| 4 | HC-SR04 Trig — Arduino Digital Pin 7 |
| 5 | HC-SR04 Echo — Arduino Digital Pin 8 |

(The slide's picture uses two different digital pins for Trig and Echo. Pins 7 and 8 are used here so that the same wiring keeps working in Experiment 29, where the Ethernet Shield takes over pins 10–13. If the instructor's Experiment_28 sketch defines different pins, either rewire to match it or change trigPin / echoPin below.)

How it works (slides 27–30): Trig is held HIGH for 10 µs, the sensor sends an 8-cycle 40 kHz ultrasonic burst, the burst bounces off the object, and Echo stays HIGH for exactly as long as the round trip took. Sound travels at 340 m/s = 0.034 cm/µs, and the wave goes to the object and back, so distance = time × 0.034 / 2.

Code — sketch name `Experiment_28`:

const int trigPin = 7;
const int echoPin = 8;
 
long duration;     // echo pulse length in microseconds
float distance;    // distance in cm
 
void setup() {
  pinMode(trigPin, OUTPUT);
  pinMode(echoPin, INPUT);
  Serial.begin(9600);
}
 
void loop() {
  // make sure Trig is LOW, then send a clean 10 microsecond HIGH pulse
  digitalWrite(trigPin, LOW);
  delayMicroseconds(2);
  digitalWrite(trigPin, HIGH);
  delayMicroseconds(10);
  digitalWrite(trigPin, LOW);
 
  // time (in microseconds) the Echo pin stays HIGH
  duration = pulseIn(echoPin, HIGH);
 
  // distance = time x speed of sound / 2   (there and back)
  distance = duration * 0.034 / 2;
 
  Serial.print("Distance: ");
  Serial.print(distance);
  Serial.println(" cm");
 
  delay(500);
}

How to observe it: Serial Monitor at 9600 baud — hold a book at 10 cm, 20 cm and 30 cm in front of the sensor and compare the readings with a ruler. The sensor is reliable roughly from 2 cm to 400 cm.

Report checklist: 28.a (video with voice-over), 28.b (duration = pulseIn(echoPin, HIGH) waits for Echo to go HIGH, starts a timer, waits for it to go LOW, and returns the HIGH time in microseconds — returns 0 if no pulse arrives before the timeout (default 1 s)), 28.c (distance equation d = t × v / 2, with v = 0.034 cm/µs in air; under water the speed of sound is about 1480 m/s ≈ 0.148 cm/µs, so the formula becomes d = t × 0.148 / 2 — about 4.35× larger per microsecond. Note the standard HC-SR04 is not waterproof; a sealed waterproof transducer would be needed).

## Experiment 29 — Send Ultrasonic Distance to ThingSpeak

Experiment 28's sensor wiring plus the Ethernet Shield. Because the shield reserves pins 10–13, the sensor stays on pins 7 and 8 (see note above).

| # | Connection |
| --- | --- |
| 1 | Seat the Ethernet Shield onto the Arduino UNO headers (all pins aligned, pressed flush) |
| 2 | Ethernet Shield RJ45 port — network cable — router with internet access |
| 3 | HC-SR04 Vcc — Arduino 5V |
| 4 | HC-SR04 Gnd — Arduino GND |
| 5 | HC-SR04 Trig — Arduino Digital Pin 7 |
| 6 | HC-SR04 Echo — Arduino Digital Pin 8 |
| 7 | Power the Arduino UNO (through the shield) from the PC using the USB cable |

Code (Experiment 28's measurement + Experiment 20's ThingSpeak upload):

#include <SPI.h>
#include <Ethernet.h>
 
byte mac[] = { 0xDE, 0xAD, 0xBE, 0xEF, 0xFE, 0xED };
EthernetClient client;
char server[] = "api.thingspeak.com";
String writeAPIKey = "YOUR_THINGSPEAK_WRITE_API_KEY";  // channel's API Keys tab
 
const int trigPin = 7;
const int echoPin = 8;
unsigned long lastUpdate = 0;
 
float measureDistanceCm() {
  digitalWrite(trigPin, LOW);
  delayMicroseconds(2);
  digitalWrite(trigPin, HIGH);
  delayMicroseconds(10);
  digitalWrite(trigPin, LOW);
  long duration = pulseIn(echoPin, HIGH);
  return duration * 0.034 / 2;
}
 
void setup() {
  Serial.begin(9600);
  pinMode(trigPin, OUTPUT);
  pinMode(echoPin, INPUT);
  Ethernet.begin(mac);                 // DHCP
}
 
void loop() {
  float distance = measureDistanceCm();
  Serial.print("Distance: ");
  Serial.print(distance);
  Serial.println(" cm");
 
  if (millis() - lastUpdate > 15000) {   // ThingSpeak free tier: 1 update / 15 s minimum
    lastUpdate = millis();
    if (client.connect(server, 80)) {
      String body = "api_key=" + writeAPIKey + "&field1=" + String(distance);
      client.println("POST /update HTTP/1.1");
      client.println("Host: api.thingspeak.com");
      client.println("Connection: close");
      client.println("Content-Type: application/x-www-form-urlencoded");
      client.print("Content-Length: ");
      client.println(body.length());
      client.println();
      client.println(body);
      client.stop();
    }
  }
  delay(1000);
}

How to observe it: move an object in front of the sensor; the Serial Monitor shows each reading, and the ThingSpeak channel's Field 1 chart gains one new point about every 15 s.

Report checklist: 29.a (video with voice-over — show the sensor, the Serial Monitor and the ThingSpeak graph), 29.b (paste the new sketch), 29.c (describe the changes vs. Experiment 28: added SPI.h/Ethernet.h, a MAC address, Ethernet.begin(mac), an EthernetClient, the ThingSpeak host and Write API key; measurement code moved into a function; an HTTP POST to /update carrying field1=<distance> every 15 s; Experiment 28's Serial printing kept for local viewing).

## Experiment 30 — 16x2 LCD Display

Wiring follows the schematic on slide 39 (LiquidCrystal "Hello World" circuit): the LCD runs in 4-bit mode (data pins DB4–DB7 only), with the contrast set by a 10 kΩ potentiometer and the backlight fed through a 220 Ω resistor.

| # | Connection |
| --- | --- |
| 1 | Power the Arduino UNO from the PC using the USB cable |
| 2 | Arduino 5V — breadboard + rail; Arduino GND — breadboard − rail |
| 3 | LCD pin 1 (VSS) — − rail (GND) |
| 4 | LCD pin 2 (VDD) — + rail (5V) |
| 5 | Potentiometer outer leg A — + rail (5V) |
| 6 | Potentiometer outer leg B — − rail (GND) |
| 7 | Potentiometer wiper (middle leg) — LCD pin 3 (V0) (contrast) |
| 8 | LCD pin 4 (RS) — Arduino Digital Pin 12 |
| 9 | LCD pin 5 (R/W) — − rail (GND) (write-only) |
| 10 | LCD pin 6 (E) — Arduino Digital Pin 11 |
| 11 | LCD pins 7–10 (DB0–DB3) — not connected |
| 12 | LCD pin 11 (DB4) — Arduino Digital Pin 5 |
| 13 | LCD pin 12 (DB5) — Arduino Digital Pin 4 |
| 14 | LCD pin 13 (DB6) — Arduino Digital Pin 3 |
| 15 | LCD pin 14 (DB7) — Arduino Digital Pin 2 |
| 16 | LCD pin 15 (LED+) — one end of the 220 Ω resistor |
| 17 | 220 Ω resistor other end — + rail (5V) |
| 18 | LCD pin 16 (LED−) — − rail (GND) |

Turn the contrast potentiometer until the characters appear (if you see only solid blocks or nothing, adjust the knob first).

Code — sketch name `Experiment_30`, Part 1 (standard LiquidCrystal "Hello World"):

#include <LiquidCrystal.h>
 
// RS, E, D4, D5, D6, D7
LiquidCrystal lcd(12, 11, 5, 4, 3, 2);
 
void setup() {
  lcd.begin(16, 2);          // 16 columns, 2 rows
  lcd.print("hello, world!");
}
 
void loop() {
  lcd.setCursor(0, 1);       // column 0, second row
  lcd.print(millis() / 1000); // seconds since reset
}

Code — Part 3 (today's date instead of the greeting):

#include <LiquidCrystal.h>
 
LiquidCrystal lcd(12, 11, 5, 4, 3, 2);
 
void setup() {
  lcd.begin(16, 2);
  lcd.setCursor(0, 0);
  lcd.print("Date: 05/10/2026");   // change to today's date
  lcd.setCursor(0, 1);
  lcd.print("IoT Experiment 30");
}
 
void loop() {
}

Report checklist: 30.a (one combined video with voice-over covering Point 1 and Point 3), 30.b (the updated sketch from Point 3), 30.c (describe the change: the text sent with lcd.print() was replaced from "hello, world!" to the date, and the running-seconds counter in loop() was replaced by a second fixed line).

## Experiment 31 — Read Data from Another Arduino's Web Server

Two Arduinos on the same network. Board A is the server — it runs the web-page sketch from Experiment 18 or 19 and is simply left powered. Board B (this experiment) is the client that connects to Board A and prints what it receives.

| # | Connection (Board B) |
| --- | --- |
| 1 | Seat the Ethernet Shield onto Board B's UNO headers |
| 2 | Board B shield RJ45 port — network cable — the same router/switch Board A is connected to |
| 3 | Power Board B from the PC using the USB cable (Serial Monitor shows the received page) |

On Board A, make sure the server's static address is the one you will use below (for the Experiment 18 sketch that is 192.168.1.177) and that it is up before starting Board B.

Code — sketch name `Experiment_31` (Arduino "WebClient" pattern):

#include <SPI.h>
#include <Ethernet.h>
 
// Board B needs its OWN MAC address (different from Board A's)
byte mac[] = { 0xDE, 0xAD, 0xBE, 0xEF, 0xFE, 0xEE };
 
IPAddress server(192, 168, 1, 177);   // Board A's IP address (the web-page server)
IPAddress ip(192, 168, 1, 178);       // a free address for Board B on the same network
 
EthernetClient client;
 
void setup() {
  Serial.begin(9600);
  Ethernet.begin(mac, ip);
  delay(1000);                         // give the shield time to start
  Serial.println("connecting...");
 
  if (client.connect(server, 80)) {
    Serial.println("connected");
    client.println("GET / HTTP/1.1");  // ask Board A for its web page
    client.println("Host: 192.168.1.177");
    client.println("Connection: close");
    client.println();
  } else {
    Serial.println("connection failed");
  }
}
 
void loop() {
  // print everything Board A sends back
  while (client.available()) {
    char c = client.read();
    Serial.print(c);
  }
 
  if (!client.connected()) {
    Serial.println();
    Serial.println("disconnecting.");
    client.stop();
    while (true);                      // stop here (press reset to fetch again)
  }
}

How to observe it: Serial Monitor at 9600 baud shows connecting... connected, then the HTTP header lines and the HTML text that Board A produces (e.g. analog input 0 is 512, or The current LDR value is ... for the Experiment 19 sketch), then disconnecting..

Report checklist: 31.a (video with voice-over — show Board A's sensors changing and Board B's Serial Monitor receiving it), 31.b (31.b.1 SPI.h and Ethernet.h; 31.b.2 byte mac[] = {...} array passed to Ethernet.begin(mac, ip); 31.b.3 an IPAddress object (ip for this board, server for the target board); 31.b.4 raw HTTP text over a TCP connection on port 80 using EthernetClient).

# 5. Quick Reference Summary

| Experiment | Input pin(s) | Output pin(s) | Extra hardware |
| --- | --- | --- | --- |
| 21 | — | 2–8 (segments a–g) | 7-segment display + 7 × 220 Ω |
| 22 | A0 (pot) | — (Serial) | Potentiometer |
| 23 | 2 (button) | — (Serial) | Push button + pull-up resistor |
| 24 | A0 (pot) | 9 (PWM LED) + Serial | Potentiometer + LED |
| 25 | A0 (pot) | 9 (PWM LED) + Serial | Same as 24 (no map()) |
| 26 | USB serial in | USB serial out | None |
| 27 | USB serial in | 3, 4, 5 (LEDs) | 3 LEDs + 3 resistors |
| 28 | 8 (Echo) | 7 (Trig) + Serial | HC-SR04 |
| 29 | 8 (Echo) | 7 (Trig); 10–13 reserved | HC-SR04 + Ethernet Shield + ThingSpeak |
| 30 | — | 12 (RS), 11 (E), 5/4/3/2 (D4–D7) | 16x2 LCD + 10 kΩ pot + 220 Ω |
| 31 | Ethernet (client) | Serial | 2 × (Arduino + Ethernet Shield) |