# Experiment Set 4 — Step-by-Step Lab Guide (Raspberry Pi)

Submission due: 20/11/2026. This set moves from the Arduino UNO to the Raspberry Pi, and every program is now written in Python instead of C. It starts with setting up the Pi itself, then repeats the LED / duty-cycle / push-button work of Set 1 on the Pi's GPIO header, and ends with three ThingSpeak uploads (ultrasonic distance, DHT22 temperature + humidity, IR obstacle sensor). Wiring is given as direct component-to-component connections — build each connection in order, landing both ends of a "connection" in the same breadboard row (or a power rail) to join them electrically.

This guide tells you how to build and run each experiment. The graded report questions from the PDF are listed at the end of each experiment as a checklist — answer those yourself (they need your own explanation, photos/video, and measurements). The slide deck does not print any programs for this set. It gives only the task and the questions. The Python programs below are standard, working RPi.GPIO implementations that match the described behaviour; the same files are saved in experiment-4/ as exp-42.py … exp-52.py. Swap in the instructor's exact file if it differs; the wiring and concepts stay the same.

# 1. Components Used

| Component | Photo | Pins / Legs |
| --- | --- | --- |
| Raspberry Pi (3B+ / 4B / 5) | (not in image set) | 40-pin GPIO header. Pin 1 is the one with the square solder pad, nearest the corner of the board (by the microSD side). Odd pins (1, 3, 5 … 39) are the inner row, even pins (2, 4 … 40) the outer row (the row closest to the board edge). |
| microSD card (16 GB or more) + card reader | (not in image set) | Holds the operating system. |
| Power supply | (not in image set) | Pi 4: 5 V 3 A USB-C. Pi 5: 5 V 5 A USB-C (27 W official supply). Pi 3B+: 5 V 2.5 A micro-USB. |
| Monitor + micro-HDMI/HDMI cable, USB keyboard, USB mouse | (not in image set) | Needed only for first set-up; afterwards SSH/VNC from the laptop is enough. |
| Breadboard | component-images/breadboard.jpeg | Top/bottom rails: red +, blue −. Center strips: rows A–E / F–J, columns 1–60; A–E not joined to F–J. |
| LED, resistor (220 Ω), jumper wires | led-light.jpeg, resistor.jpeg, wire.jpeg | LED: long leg = anode (+), short leg = cathode (−). Use female-to-male jumpers from the Pi header to the breadboard. |
| Push button switch (×2) + 10 kΩ resistor (×2) | component-images/switch.jpeg | 4 legs: the 2 legs on the same side of the body are always joined; the two sides join only while the button is pressed. |
| HC-SR04 ultrasonic sensor | component-images/ultrasonic-sensor.jpeg | Vcc, Trig, Echo, Gnd. Runs on 5 V, so its Echo output is 5 V — it must go through a 1 kΩ / 2 kΩ voltage divider before it reaches the Pi (Pi GPIO pins are 3.3 V only). |
| DHT22 (AM2302) temperature & humidity sensor | (not in image set) | Bare sensor: 4 pins, left to right with the grille facing you: 1 VCC, 2 DATA, 3 NC, 4 GND (needs a 10 kΩ pull-up from DATA to VCC). 3-pin module: +, OUT, − (pull-up already on the board). |
| IR obstacle sensor module (FC-51 type) | component-images/infrared-sensor.jpeg | 3 pins: VCC, GND, OUT. OUT goes LOW when an object is in front of it. The blue trimmer sets the detection distance (about 2–30 cm). |

3.3 V rule. Raspberry Pi GPIO pins work at 3.3 V and are not 5 V tolerant. Never connect a 5 V signal directly to a GPIO pin — it can permanently damage the Pi. Each pin should not supply more than about 16 mA.

# 2. Raspberry Pi Header — Pins Used in This Set

The PDF numbers pins by their physical position on the header (Pin 11, 13, 15 …), so every program uses GPIO.setmode(GPIO.BOARD). The BCM (GPIO) names are given only for reference.

| Physical pin | Name | Used in |
| --- | --- | --- |
| 1 | 3.3 V | Exp 44 (LED 2), 48–49 (switches), 51 (DHT22), 52 (IR) |
| 2 | 5 V | Exp 50 (HC-SR04 Vcc) |
| 6 | GND | All experiments |
| 7 | GPIO4 | Exp 51 (DHT22 DATA) |
| 9, 14, 20, 25, 30, 34, 39 | GND | Extra ground pins |
| 11 | GPIO17 | LED 1 (Exp 42–49) |
| 12 | GPIO18 | Exp 52 (IR OUT) |
| 13 | GPIO27 | LED 2 (Exp 45–49) |
| 15 | GPIO22 | LED 3 (Exp 45–49) |
| 16 | GPIO23 | Exp 50 (HC-SR04 Trig) |
| 17 | 3.3 V | Second 3.3 V pin |
| 18 | GPIO24 | Exp 50 (HC-SR04 Echo, via divider) |
| 29 | GPIO5 | Switch 1 (Exp 48–49) |
| 31 | GPIO6 | Switch 2 (Exp 48–49) |

Breadboard rails: Pi pin 6 (GND) → blue − rail, Pi pin 1 (3.3 V) → red + rail (only when the experiment needs 3.3 V on the breadboard). Do not put 5 V on the same rail.

# 3. One-Time Software Set-up (this is also Experiment 41)

See Experiment 41 below for the full step list. After set-up, every program in this set is run the same way:

| Step | From | To |
| --- | --- | --- |
| 1 | Raspberry Pi desktop | Open Thonny (Menu → Programming → Thonny) |
| 2 | Thonny | Paste the program and Save as e.g. exp-42.py in /home/<user>/iot/ |
| 3 | Thonny | Press Run (F5). The output (prints) appears in the Shell pane at the bottom |
| 4 | Thonny | Press Stop (Ctrl+F2) to end the program — GPIO.cleanup() then turns all pins off |

Or from a terminal: python3 exp-42.py, and Ctrl+C to stop.

# 4. Experiment-by-Experiment Build

## Experiment 41 — Set-up of the Raspberry Pi

No circuit. These steps take a new Pi to a working desktop with Python and GPIO ready.

| # | Step |
| --- | --- |
| 1 | On the laptop, install Raspberry Pi Imager from raspberrypi.com/software |
| 2 | Put the microSD card into the card reader and plug it into the laptop |
| 3 | In Imager: Choose Device → your Pi model; Choose OS → Raspberry Pi OS (64-bit) (with desktop); Choose Storage → the microSD card |
| 4 | Click Next → Edit Settings (OS customisation): set the hostname (e.g. iotpi), username + password, Wi-Fi SSID + password + country (IN), time zone (Asia/Kolkata) and keyboard layout. On the Services tab tick Enable SSH (password authentication) |
| 5 | Click Save → Yes → Yes to write the card. Wait for Write Successful, then eject the card |
| 6 | Insert the microSD card into the Pi's card slot (underside of the board) |
| 7 | Connect the monitor (HDMI / micro-HDMI port 0), the USB keyboard and the USB mouse. Connect a LAN cable if you are not using Wi-Fi |
| 8 | Connect the power supply last. The red LED stays on and the green LED flickers while it boots; the first boot takes a few minutes and may reboot once by itself (it expands the file system) |
| 9 | The desktop appears. If you skipped customisation, the Welcome to Raspberry Pi wizard asks for country, user, password and Wi-Fi now |
| 10 | Open a Terminal and update the system: sudo apt update then sudo apt full-upgrade -y, then sudo reboot |
| 11 | Find the Pi's IP address with hostname -I (needed for SSH/VNC) |
| 12 | Run sudo raspi-config → Interface Options → enable SSH and VNC if you want to use the Pi from the laptop. (Or use Menu → Preferences → Raspberry Pi Configuration → Interfaces) |
| 13 | Check Python: python3 --version (Python 3 is already installed) and that Thonny is in Menu → Programming |
| 14 | Check the GPIO library: python3 -c "import RPi.GPIO" should print nothing (no error). On a Pi 5 run sudo apt install python3-rpi-lgpio first — it is a drop-in replacement, so the same import RPi.GPIO as GPIO programs work |
| 15 | Install the libraries for Experiments 50–52: sudo apt install python3-requests |
| 16 | For the DHT22 (Experiment 51) make a virtual environment, because new Raspberry Pi OS blocks system-wide pip: python3 -m venv --system-site-packages ~/iot-env, then source ~/iot-env/bin/activate, then pip install adafruit-circuitpython-dht. In Thonny select this interpreter via Run → Configure interpreter → Python executable → ~/iot-env/bin/python3 |
| 17 | Test the header: run pinout in the terminal — it prints the 40-pin map of your exact board |
| 18 | Shut down properly before unplugging: sudo shutdown -h now (wait for the green LED to stop) |

From the laptop you can now log in with ssh <user>@iotpi.local (or the IP from step 11) or use RealVNC Viewer to see the desktop.

Report checklist: 41.a (describe all the set-up steps in detail — use the table above, and add screenshots of the Imager settings and the first desktop).

## Experiment 42 — Blink One LED on Pin 11 (1 s ON, 1 s OFF)

| # | Connection |
| --- | --- |
| 1 | Pi physical pin 11 (GPIO17) — one end of the 220 Ω resistor |
| 2 | Resistor other end — LED anode (long leg) |
| 3 | LED cathode (short leg) — Pi physical pin 6 (GND) |
| 4 | Power the Pi and open Thonny |

(LED current = (3.3 V − 2 V) / 220 Ω ≈ 6 mA, well inside the Pi pin limit.)

Code — experiment-4/exp-42.py:

import RPi.GPIO as GPIO
import time
 
LED_PIN = 11                     # physical pin 11 (GPIO17)
 
GPIO.setwarnings(False)
GPIO.setmode(GPIO.BOARD)         # use physical pin numbers
GPIO.setup(LED_PIN, GPIO.OUT, initial=GPIO.LOW)
 
try:
    while True:
        GPIO.output(LED_PIN, GPIO.HIGH)   # LED ON
        time.sleep(1)
        GPIO.output(LED_PIN, GPIO.LOW)    # LED OFF
        time.sleep(1)
except KeyboardInterrupt:
    pass
finally:
    GPIO.cleanup()

Report checklist: 42.a (video with voice-over), 42.b (paste the code).

## Experiment 43 — 20 % Duty Cycle on Pin 11

Hardware is identical to Experiment 42.

| # | Connection |
| --- | --- |
| 1 | Pi pin 11 — 220 Ω resistor — LED anode |
| 2 | LED cathode — Pi pin 6 (GND) |

Code — experiment-4/exp-43.py (period kept at 1 s: ON 0.2 s, OFF 0.8 s):

import RPi.GPIO as GPIO
import time
 
LED_PIN = 11                     # physical pin 11 (GPIO17)
 
GPIO.setwarnings(False)
GPIO.setmode(GPIO.BOARD)
GPIO.setup(LED_PIN, GPIO.OUT, initial=GPIO.LOW)
 
try:
    while True:
        GPIO.output(LED_PIN, GPIO.HIGH)   # ON for 20% of the 1 s period
        time.sleep(0.2)
        GPIO.output(LED_PIN, GPIO.LOW)    # OFF for 80% of the 1 s period
        time.sleep(0.8)
except KeyboardInterrupt:
    pass
finally:
    GPIO.cleanup()

Duty cycle = ON time / (ON + OFF) = 0.2 / 1.0 = 20 %.

Report checklist: 43.a (video with voice-over), 43.b (paste the program), 43.c (changes vs. 42: time.sleep(1) after HIGH changed to 0.2, and time.sleep(1) after LOW changed to 0.8; the period stays 1 s, nothing else changed).

## Experiment 44 — Two LEDs, 20 % and 80 %, Same Program as 43

The program of Experiment 43 is not changed — run exp-43.py again. Only the hardware changes: LED 2 is connected upside down to the same pin 11, between 3.3 V and the pin. When pin 11 is HIGH (3.3 V) both ends of LED 2 are at 3.3 V, so it is OFF; when pin 11 is LOW (0 V) current flows from 3.3 V through LED 2 into pin 11, so it is ON. LED 2 therefore does exactly the opposite of LED 1: ON for 80 %, OFF for 20 %.

| # | Connection |
| --- | --- |
| 1 | (LED 1, unchanged) Pi pin 11 — Resistor 1 — LED 1 anode |
| 2 | (LED 1, unchanged) LED 1 cathode — Pi pin 6 (GND) |
| 3 | Pi pin 1 (3.3 V) — one end of Resistor 2 (220 Ω) |
| 4 | Resistor 2 other end — LED 2 anode (long leg) |
| 5 | LED 2 cathode (short leg) — Pi pin 11 (same row as Resistor 1's pin-11 end) |
| 6 | Run exp-43.py without editing it |

Use 3.3 V (pin 1), not 5 V, for LED 2. With 5 V, LED 2 would push 5 V into pin 11 while it is HIGH, which can damage the Pi.

Report checklist: 44.a (video with voice-over — show both LEDs: LED 1 short flash, LED 2 long ON), 44.b (describe the hardware changes: second LED + 220 Ω added between 3.3 V and pin 11, with its cathode on pin 11, so it lights when the pin is LOW — "active-low" connection; program untouched).

## Experiment 45 — Blink LEDs on Pins 11, 13 and 15 Together

| # | Connection |
| --- | --- |
| 1 | Pi pin 6 (GND) — breadboard blue − rail |
| 2 | Pi pin 11 — Resistor 1 (220 Ω) — LED 1 anode; LED 1 cathode — − rail |
| 3 | Pi pin 13 — Resistor 2 (220 Ω) — LED 2 anode; LED 2 cathode — − rail |
| 4 | Pi pin 15 — Resistor 3 (220 Ω) — LED 3 anode; LED 3 cathode — − rail |

(If LED 2 from Experiment 44 is still connected to pin 11 and 3.3 V, remove it first.)

Code — experiment-4/exp-45.py:

import RPi.GPIO as GPIO
import time
 
LEDS = [11, 13, 15]              # GPIO17, GPIO27, GPIO22
 
GPIO.setwarnings(False)
GPIO.setmode(GPIO.BOARD)
GPIO.setup(LEDS, GPIO.OUT, initial=GPIO.LOW)
 
try:
    while True:
        GPIO.output(LEDS, GPIO.HIGH)      # all three ON together
        time.sleep(1)
        GPIO.output(LEDS, GPIO.LOW)       # all three OFF together
        time.sleep(1)
except KeyboardInterrupt:
    pass
finally:
    GPIO.cleanup()

GPIO.setup() and GPIO.output() accept a list of pins, so one call switches all three at once.

Report checklist: 45.a (video with voice-over), 45.b (paste the program).

## Experiment 46 — Turn On 11 → 13 → 15 One After Another (2 s), Then All OFF

Hardware is identical to Experiment 45.

| # | Connection |
| --- | --- |
| 1 | Pi pin 6 (GND) — − rail |
| 2 | Pi pins 11 / 13 / 15 — each through its own 220 Ω — LED anode; each cathode — − rail |

Code — experiment-4/exp-46.py:

import RPi.GPIO as GPIO
import time
 
LEDS = [11, 13, 15]
 
GPIO.setwarnings(False)
GPIO.setmode(GPIO.BOARD)
GPIO.setup(LEDS, GPIO.OUT, initial=GPIO.LOW)
 
try:
    while True:
        for pin in LEDS:                  # 11, then 13, then 15
            GPIO.output(pin, GPIO.HIGH)   # this LED stays ON
            time.sleep(2)
        GPIO.output(LEDS, GPIO.LOW)       # all OFF together
        time.sleep(2)
except KeyboardInterrupt:
    pass
finally:
    GPIO.cleanup()

Timeline: 0 s LED 11 ON → 2 s LED 13 ON → 4 s LED 15 ON → 6 s all OFF → 8 s repeat.

Report checklist: 46.a (video with voice-over), 46.b (paste the program).

## Experiment 47 — Rotating Single LED (11 → 13 → 15 → 11 …), 2 s Each

Hardware is identical to Experiment 45.

| # | Connection |
| --- | --- |
| 1 | Pi pin 6 (GND) — − rail |
| 2 | Pi pins 11 / 13 / 15 — each through its own 220 Ω — LED anode; each cathode — − rail |

Code — experiment-4/exp-47.py:

import RPi.GPIO as GPIO
import time
 
LEDS = [11, 13, 15]
 
GPIO.setwarnings(False)
GPIO.setmode(GPIO.BOARD)
GPIO.setup(LEDS, GPIO.OUT, initial=GPIO.LOW)
 
try:
    while True:
        for pin in LEDS:
            GPIO.output(LEDS, GPIO.LOW)   # turn every LED off
            GPIO.output(pin, GPIO.HIGH)   # then turn only this one on
            time.sleep(2)
except KeyboardInterrupt:
    pass
finally:
    GPIO.cleanup()

Report checklist: 47.a (video with voice-over), 47.b (paste the program — save it as a new file).

## Experiment 48 — Two Push Buttons (Pins 29, 31) Control the LEDs

Pressing a switch must send logic 1 to the Pi, so each switch goes between 3.3 V and the input pin, and a 10 kΩ pull-down resistor holds the pin at 0 V while the switch is open. (The program also turns on the Pi's internal pull-down, PUD_DOWN, as a backup.)

| # | Connection |
| --- | --- |
| 1 | Pi pin 6 (GND) — blue − rail; Pi pin 1 (3.3 V) — red + rail |
| 2 | LEDs as in Experiment 45: pins 11 / 13 / 15 — 220 Ω — LED anode; cathodes — − rail |
| 3 | Switch 1 Side A — + rail (3.3 V) |
| 4 | Switch 1 Side B — Pi pin 29 (GPIO5) (this row is the junction) |
| 5 | Switch 1 junction (step 4) — 10 kΩ resistor — − rail (GND) |
| 6 | Switch 2 Side A — + rail (3.3 V) |
| 7 | Switch 2 Side B — Pi pin 31 (GPIO6) (junction) |
| 8 | Switch 2 junction (step 7) — 10 kΩ resistor — − rail (GND) |

Use 3.3 V, never 5 V, for the switches.

Code — experiment-4/exp-48.py:

import RPi.GPIO as GPIO
import time
 
LED1, LED2, LED3 = 11, 13, 15
SW1, SW2 = 29, 31                # GPIO5, GPIO6
 
GPIO.setwarnings(False)
GPIO.setmode(GPIO.BOARD)
GPIO.setup([LED1, LED2, LED3], GPIO.OUT, initial=GPIO.LOW)
# switch gives 3.3 V (logic 1) when pressed, pull-down keeps it at 0 otherwise
GPIO.setup([SW1, SW2], GPIO.IN, pull_up_down=GPIO.PUD_DOWN)
 
try:
    while True:
        s1 = GPIO.input(SW1)
        s2 = GPIO.input(SW2)
 
        GPIO.output(LED1, s1)             # SW1 pressed -> LED on pin 11
        GPIO.output(LED2, s2)             # SW2 pressed -> LEDs on 13 and 15
        GPIO.output(LED3, s2)
 
        time.sleep(0.05)
except KeyboardInterrupt:
    pass
finally:
    GPIO.cleanup()

| Switch 29 | Switch 31 | LED 11 | LED 13 | LED 15 |
| --- | --- | --- | --- | --- |
| released | released | OFF | OFF | OFF |
| pressed | released | ON | OFF | OFF |
| released | pressed | OFF | ON | ON |
| pressed | pressed | ON | ON | ON |

Report checklist: 48.a (video with voice-over — press each switch, then both), 48.b (paste the program).

## Experiment 49 — Inverted: All LEDs ON, Pressed Switch Turns Them OFF

Hardware is identical to Experiment 48.

| # | Connection |
| --- | --- |
| 1 | Same LEDs on pins 11 / 13 / 15 and same switches on pins 29 / 31 (3.3 V + 10 kΩ pull-down) as Experiment 48 |

Code — experiment-4/exp-49.py:

import RPi.GPIO as GPIO
import time
 
LED1, LED2, LED3 = 11, 13, 15
SW1, SW2 = 29, 31
 
GPIO.setwarnings(False)
GPIO.setmode(GPIO.BOARD)
GPIO.setup([LED1, LED2, LED3], GPIO.OUT, initial=GPIO.HIGH)   # all ON at start
GPIO.setup([SW1, SW2], GPIO.IN, pull_up_down=GPIO.PUD_DOWN)
 
try:
    while True:
        s1 = GPIO.input(SW1)
        s2 = GPIO.input(SW2)
 
        # pressed (1) -> LED OFF, released (0) -> LED ON
        GPIO.output(LED1, GPIO.LOW if s1 else GPIO.HIGH)
        GPIO.output(LED2, GPIO.LOW if s2 else GPIO.HIGH)
        GPIO.output(LED3, GPIO.LOW if s2 else GPIO.HIGH)
 
        time.sleep(0.05)
except KeyboardInterrupt:
    pass
finally:
    GPIO.cleanup()

Report checklist: 49.a (video with voice-over), 49.b (paste the program), 49.c (changes vs. 48: initial=GPIO.LOW → initial=GPIO.HIGH so all LEDs start ON; each GPIO.output(LED, s) replaced by GPIO.output(LED, GPIO.LOW if s else GPIO.HIGH), i.e. the switch value is inverted before it is written; hardware unchanged).

## Experiment 50 — Ultrasonic Sensor → ThingSpeak

The HC-SR04 needs 5 V, and its Echo pin outputs 5 V, so Echo goes through a voltage divider (1 kΩ on top, 2 kΩ to ground) which gives 5 × 2 / (1 + 2) ≈ 3.3 V at the Pi pin. If you have no 2 kΩ resistor, use two 1 kΩ in series.

| # | Connection |
| --- | --- |
| 1 | Pi pin 6 (GND) — blue − rail |
| 2 | HC-SR04 Vcc — Pi pin 2 (5 V) |
| 3 | HC-SR04 Gnd — − rail |
| 4 | HC-SR04 Trig — Pi pin 16 (GPIO23) (3.3 V from the Pi is enough to trigger it) |
| 5 | HC-SR04 Echo — one end of the 1 kΩ resistor |
| 6 | 1 kΩ other end — a free breadboard row (the divider junction) |
| 7 | Junction — 2 kΩ resistor — − rail (GND) |
| 8 | Junction — Pi pin 18 (GPIO24) |
| 9 | Pi connected to the internet (Wi-Fi or LAN) |

ThingSpeak: log in at thingspeak.mathworks.com → Channels → New Channel → name it (e.g. Pi Ultrasonic), tick Field 1 = Distance (cm) → Save. Copy the Write API Key from the API Keys tab into API_KEY below.

Code — experiment-4/exp-50.py:

import RPi.GPIO as GPIO
import time
import requests
 
API_KEY = "YOUR_WRITE_API_KEY"            # ThingSpeak channel -> API Keys tab
URL = "https://api.thingspeak.com/update"
 
TRIG = 16                                 # GPIO23
ECHO = 18                                 # GPIO24 (through the 1k/2k divider)
 
GPIO.setwarnings(False)
GPIO.setmode(GPIO.BOARD)
GPIO.setup(TRIG, GPIO.OUT, initial=GPIO.LOW)
GPIO.setup(ECHO, GPIO.IN)
time.sleep(2)                             # let the sensor settle
 
 
def get_distance():
    # 10 microsecond pulse on Trig starts one measurement
    GPIO.output(TRIG, GPIO.HIGH)
    time.sleep(0.00001)
    GPIO.output(TRIG, GPIO.LOW)
 
    timeout = time.time() + 0.05
    start = time.time()
    while GPIO.input(ECHO) == 0:          # wait for Echo to go HIGH
        start = time.time()
        if start > timeout:
            return None
    end = time.time()
    while GPIO.input(ECHO) == 1:          # wait for Echo to go LOW again
        end = time.time()
        if end > timeout:
            return None
 
    duration = end - start                # round trip time in seconds
    return round(duration * 34300 / 2, 2) # speed of sound 34300 cm/s
 
 
last_upload = 0
 
try:
    while True:
        distance = get_distance()
        if distance is None:
            print("No echo received")
        else:
            print("Distance:", distance, "cm")
 
            if time.time() - last_upload >= 15:   # free ThingSpeak: 1 update / 15 s
                try:
                    r = requests.get(URL, params={"api_key": API_KEY,
                                                  "field1": distance}, timeout=10)
                    print("Sent to ThingSpeak, entry:", r.text)
                    last_upload = time.time()
                except requests.RequestException as e:
                    print("Upload failed:", e)
        time.sleep(1)
except KeyboardInterrupt:
    pass
finally:
    GPIO.cleanup()

How to observe it: Thonny's Shell prints the distance every second; every 15 s it also prints Sent to ThingSpeak, entry: N (N = entry number; 0 means the update was rejected, usually a wrong API key or sending faster than 15 s). Open the channel's Private View — the Field 1 chart gains one point every 15 s. Move a book closer / farther to see the line change.

Report checklist: 50.a (video with voice-over — sensor, Shell output and the ThingSpeak chart), 50.b (paste the program).

## Experiment 51 — DHT22 Temperature & Humidity → ThingSpeak

| # | Connection (bare 4-pin DHT22) |
| --- | --- |
| 1 | DHT22 pin 1 (VCC) — Pi pin 1 (3.3 V) |
| 2 | DHT22 pin 2 (DATA) — Pi pin 7 (GPIO4) |
| 3 | 10 kΩ resistor between DHT22 pin 2 (DATA) and pin 1 (VCC) (pull-up) |
| 4 | DHT22 pin 3 — not connected |
| 5 | DHT22 pin 4 (GND) — Pi pin 6 (GND) |

(3-pin module: + → pin 1, OUT → pin 7, − → pin 6; no extra resistor.)

Library: the virtual environment from Experiment 41 step 16 (adafruit-circuitpython-dht). Run the program with that interpreter.

ThingSpeak: new channel (e.g. Pi DHT22) with Field 1 = Temperature (°C) and Field 2 = Humidity (%).

Code — experiment-4/exp-51.py:

import time
import requests
import board
import adafruit_dht
 
API_KEY = "YOUR_WRITE_API_KEY"
URL = "https://api.thingspeak.com/update"
 
dht = adafruit_dht.DHT22(board.D4)        # DATA on physical pin 7 (GPIO4)
 
try:
    while True:
        try:
            temperature = dht.temperature
            humidity = dht.humidity
        except RuntimeError as e:
            # DHT22 misses a reading now and then, just try again
            print("Reading failed:", e.args[0])
            time.sleep(2)
            continue
 
        if temperature is None or humidity is None:
            time.sleep(2)
            continue
 
        print("Temperature: {:.1f} C   Humidity: {:.1f} %".format(temperature, humidity))
 
        try:
            r = requests.get(URL, params={"api_key": API_KEY,
                                          "field1": temperature,
                                          "field2": humidity}, timeout=10)
            print("Sent to ThingSpeak, entry:", r.text)
        except requests.RequestException as e:
            print("Upload failed:", e)
 
        time.sleep(15)                    # ThingSpeak free limit
except KeyboardInterrupt:
    pass
finally:
    dht.exit()

How to observe it: the Shell prints temperature and humidity every 15 s (an occasional Reading failed: Checksum did not validate is normal for the DHT22 — the program just retries). Breathe on the sensor or hold it in your hand: humidity and temperature rise on both charts.

Report checklist: 51.a (video with voice-over — show both Field 1 and Field 2 charts), 51.b (paste the program).

## Experiment 52 — IR Obstacle Sensor → ThingSpeak

| # | Connection |
| --- | --- |
| 1 | IR module VCC — Pi pin 1 (3.3 V) (powering it from 3.3 V keeps its OUT signal at 3.3 V) |
| 2 | IR module GND — Pi pin 6 (GND) |
| 3 | IR module OUT — Pi pin 12 (GPIO18) |

Turn the blue trimmer on the module until its obstacle LED lights when your hand is about 10 cm away and goes off when you take it away.

ThingSpeak: new channel (e.g. Pi IR Sensor) with Field 1 = Object detected (1/0).

Code — experiment-4/exp-52.py:

import RPi.GPIO as GPIO
import time
import requests
 
API_KEY = "YOUR_WRITE_API_KEY"
URL = "https://api.thingspeak.com/update"
 
IR_PIN = 12                               # GPIO18, module OUT pin
 
GPIO.setwarnings(False)
GPIO.setmode(GPIO.BOARD)
GPIO.setup(IR_PIN, GPIO.IN)
 
last_upload = 0
 
try:
    while True:
        # module output is LOW when an object is in front of it
        detected = 1 if GPIO.input(IR_PIN) == GPIO.LOW else 0
        print("Object detected" if detected else "No object")
 
        if time.time() - last_upload >= 15:
            try:
                r = requests.get(URL, params={"api_key": API_KEY,
                                              "field1": detected}, timeout=10)
                print("Sent to ThingSpeak, entry:", r.text)
                last_upload = time.time()
            except requests.RequestException as e:
                print("Upload failed:", e)
 
        time.sleep(0.5)
except KeyboardInterrupt:
    pass
finally:
    GPIO.cleanup()

How to observe it: the Shell prints Object detected / No object twice a second. The ThingSpeak chart is a step graph that sits at 1 while an object is in front of the sensor during an upload and at 0 otherwise (hold your hand there for at least 15 s to be sure it is caught).

Report checklist: 52.a (video with voice-over), 52.b (paste the program).

# 5. Quick Reference Summary

| Experiment | Input pin(s) | Output pin(s) | Extra hardware |
| --- | --- | --- | --- |
| 41 | — | — | microSD, power supply, monitor, keyboard, mouse |
| 42 | — | 11 | 1 LED + 220 Ω |
| 43 | — | 11 (20 % duty) | Same as 42 |
| 44 | — | 11 (drives LED 1 high-side, LED 2 low-side from 3.3 V) | 2 LEDs + 2 × 220 Ω |
| 45 | — | 11, 13, 15 (together) | 3 LEDs + 3 × 220 Ω |
| 46 | — | 11 → 13 → 15, then all off | Same as 45 |
| 47 | — | 11 → 13 → 15 (one at a time) | Same as 45 |
| 48 | 29, 31 (switches, active HIGH) | 11 / 13 + 15 | 3 LEDs + 2 switches + 2 × 10 kΩ |
| 49 | 29, 31 | 11 / 13 + 15 (inverted) | Same as 48 |
| 50 | 18 (Echo via 1k/2k divider) | 16 (Trig) + ThingSpeak | HC-SR04 |
| 51 | 7 (DHT22 DATA) | ThingSpeak (Field 1 + 2) | DHT22 + 10 kΩ pull-up |
| 52 | 12 (IR OUT, active LOW) | ThingSpeak | IR obstacle module |