IoT Experiment Set 3

Circuit Diagrams — Experiments 21 to 31

Arduino UNO · 7-Segment Display · Serial Monitor/Plotter · HC-SR04 · 16x2 LCD · Ethernet Shield / ThingSpeak

Vedant Solanki

# How to Read the Diagrams

Red wire / +5V tag: Arduino 5V supply (breadboard + rail).

Black wire / GND tag: Arduino GND (breadboard − rail).

Coloured wires: signal lines. Each digital or analog pin has its own colour.

Grey thick line: USB cable to the PC (power, upload and Serial Monitor). Green-teal thick line: Ethernet cable.

A black dot marks a real electrical junction. Wires that cross without a dot are not connected.

× marks a pin that is left unconnected.

All LEDs and display segments use a 220 Ω series resistor. The LED's long leg (+, anode) faces the Arduino pin.

## Summary of Pins Used

| Exp | Input pin(s) | Output pin(s) | Extra hardware |
| --- | --- | --- | --- |
| 21 | — | D2–D8 (segments a–g) | 7-segment display + 7 × 220 Ω |
| 22 | A0 (pot) | Serial | Potentiometer |
| 23 | D2 (button) | Serial | Push button + pull-up resistor |
| 24 | A0 (pot) | D9 PWM LED + Serial | Potentiometer + LED |
| 25 | A0 (pot) | D9 PWM LED + Serial | Same as 24 (no map()) |
| 26 | USB serial in | USB serial out | None |
| 27 | USB serial in | D3, D4, D5 (LEDs) | 3 LEDs + 3 × 220 Ω |
| 28 | D8 (Echo) | D7 (Trig) + Serial | HC-SR04 |
| 29 | D8 (Echo) | D7 (Trig); D10–D13 reserved | HC-SR04 + Ethernet Shield |
| 30 | — | D12 RS, D11 E, D5–D2 DB4–DB7 | 16x2 LCD + 10 kΩ pot + 220 Ω |
| 31 | Ethernet (client) | Serial | 2 × (UNO + Ethernet Shield) |

# Experiment 21 — 7-Segment Display Counting 0 to 9

A single common-cathode 7-segment display counts from 0 to 9, changing every 2 seconds. Each segment a–g is driven by its own digital pin (D2–D8) through its own 220 Ω resistor, and the common pin goes to GND.

Figure 1: Circuit diagram of Experiment 21

## Connections

| # | Connection |
| --- | --- |
| 1 | Power the Arduino UNO from the PC using the USB cable |
| 2 | Arduino Digital Pin 2 — one end of Resistor 1 |
| 3 | Resistor 1 other end — display pin 7 (segment a) |
| 4 | Arduino Digital Pin 3 — one end of Resistor 2 |
| 5 | Resistor 2 other end — display pin 6 (segment b) |
| 6 | Arduino Digital Pin 4 — one end of Resistor 3 |
| 7 | Resistor 3 other end — display pin 4 (segment c) |
| 8 | Arduino Digital Pin 5 — one end of Resistor 4 |
| 9 | Resistor 4 other end — display pin 2 (segment d) |
| 10 | Arduino Digital Pin 6 — one end of Resistor 5 |
| 11 | Resistor 5 other end — display pin 1 (segment e) |
| 12 | Arduino Digital Pin 7 — one end of Resistor 6 |
| 13 | Resistor 6 other end — display pin 9 (segment f) |
| 14 | Arduino Digital Pin 8 — one end of Resistor 7 |
| 15 | Resistor 7 other end — display pin 10 (segment g) |
| 16 | Display pin 3 or 8 (common) — Arduino GND |

Note: If the display is common-anode, connect the COM pin to +5V and invert the outputs in the sketch with !bitRead(...).

# Experiment 22 — Potentiometer Value on Serial Monitor / Plotter

The potentiometer acts as a voltage divider between 5V and GND. Its wiper feeds A0, and the 0–1023 reading is sent over USB to the Serial Monitor and the Serial Plotter.

Figure 2: Circuit diagram of Experiment 22

## Connections

| # | Connection |
| --- | --- |
| 1 | Power the Arduino UNO from the PC using the USB cable |
| 2 | Potentiometer outer leg A — Arduino 5V |
| 3 | Potentiometer outer leg B — Arduino GND |
| 4 | Potentiometer wiper (middle leg) — Arduino A0 |

Note: The Serial Monitor and the Serial Plotter cannot be open at the same time. Set the baud rate to 9600.

# Experiment 23 — Push Button State on Serial Monitor / Plotter

A pull-up resistor keeps D2 HIGH (1) while the button is released. Pressing the button pulls the junction to GND (0).

Figure 3: Circuit diagram of Experiment 23

## Connections

| # | Connection |
| --- | --- |
| 1 | Power the Arduino UNO from the PC using the USB cable |
| 2 | Resistor (10 kΩ recommended, 220 Ω also works) — one end to Arduino 5V |
| 3 | Resistor other end — switch Side A (its two same-side legs, internally joined) (this row is the junction) |
| 4 | Switch Side B (its two same-side legs, internally joined) — Arduino GND |
| 5 | Junction of resistor and switch (step 3) — Arduino Digital Pin 2 |

Note: The junction of the resistor and the button is the only point wired to D2.

# Experiment 24 — Potentiometer → Serial Monitor + LED Brightness (map)

The potentiometer on A0 sets the PWM duty cycle of the LED on D9 (outputValue = map(sensorValue, 0, 1023, 0, 255)). Both values are printed to the Serial Monitor.

Figure 4: Circuit diagram of Experiment 24

## Connections

| # | Connection |
| --- | --- |
| 1 | Power the Arduino UNO from the PC using the USB cable |
| 2 | LED anode (long leg) — one end of the resistor |
| 3 | Resistor other end — Arduino Digital Pin 9 (9~, PWM) |
| 4 | LED cathode (short leg) — Arduino GND |
| 5 | Potentiometer outer leg A — Arduino 5V |
| 6 | Potentiometer outer leg B — Arduino GND |
| 7 | Potentiometer wiper (middle leg) — Arduino A0 |

Note: D9 is a PWM pin (~). The LED needs its 220 Ω series resistor.

# Experiment 25 — Same Circuit as 24, Without map()

The hardware is identical to Experiment 24. The sketch scales the reading with outputValue = sensorValue / 4 instead of map().

Figure 5: Circuit diagram of Experiment 25

## Connections

| # | Connection |
| --- | --- |
| 1 | Power the Arduino UNO from the PC using the USB cable |
| 2 | LED anode (long leg) — one end of the resistor |
| 3 | Resistor other end — Arduino Digital Pin 9 (PWM) |
| 4 | LED cathode (short leg) — Arduino GND |
| 5 | Potentiometer outer leg A — Arduino 5V |
| 6 | Potentiometer outer leg B — Arduino GND |
| 7 | Potentiometer wiper (middle leg) — Arduino A0 |

Note: No wiring changes from Experiment 24.

# Experiment 26 — Serial Read (Typing to the Arduino)

No external components. A character typed in the Serial Monitor is sent over USB, and the Arduino replies with its ASCII code.

Figure 6: Circuit diagram of Experiment 26

## Connections

| # | Connection |
| --- | --- |
| 1 | Power the Arduino UNO from the PC using the USB cable |

Note: With the line ending set to Newline, an extra 'I received: 10' appears for the '\n' character.

# Experiment 27 — Typed Number 3, 4 or 5 Lights That LED for 2 s

Three LEDs with 220 Ω resistors sit on D3, D4 and D5. The number typed in the Serial Monitor selects which pin goes HIGH for 2 seconds.

Figure 7: Circuit diagram of Experiment 27

## Connections

| # | Connection |
| --- | --- |
| 1 | Power the Arduino UNO from the PC using the USB cable |
| 2 | LED 1 anode (long leg) — Resistor 1 one end |
| 3 | Resistor 1 other end — Arduino Digital Pin 3 |
| 4 | LED 1 cathode (short leg) — Arduino GND |
| 5 | LED 2 anode (long leg) — Resistor 2 one end |
| 6 | Resistor 2 other end — Arduino Digital Pin 4 |
| 7 | LED 2 cathode (short leg) — Arduino GND |
| 8 | LED 3 anode (long leg) — Resistor 3 one end |
| 9 | Resistor 3 other end — Arduino Digital Pin 5 |
| 10 | LED 3 cathode (short leg) — Arduino GND |

Note: Any other input prints "Number is out of Range."

# Experiment 28 — Distance Measurement with HC-SR04

Trig (D7) receives a 10 µs pulse. Echo (D8) stays HIGH for the round-trip time of the 40 kHz burst, so distance = duration × 0.034 / 2 cm.

Figure 8: Circuit diagram of Experiment 28

## Connections

| # | Connection |
| --- | --- |
| 1 | Power the Arduino UNO from the PC using the USB cable |
| 2 | HC-SR04 Vcc — Arduino 5V |
| 3 | HC-SR04 Gnd — Arduino GND |
| 4 | HC-SR04 Trig — Arduino Digital Pin 7 |
| 5 | HC-SR04 Echo — Arduino Digital Pin 8 |

Note: Pins 7 and 8 are used so that the same wiring still works in Experiment 29, where the Ethernet Shield reserves D10–D13.

# Experiment 29 — Ultrasonic Distance Sent to ThingSpeak

This is Experiment 28's sensor wiring with the W5100 Ethernet Shield stacked on the UNO. The measured distance is uploaded to ThingSpeak Field 1 through the router.

Figure 9: Circuit diagram of Experiment 29

## Connections

| # | Connection |
| --- | --- |
| 1 | Seat the Ethernet Shield onto the Arduino UNO headers (all pins aligned, pressed flush) |
| 2 | Ethernet Shield RJ45 port — network cable — router with internet access |
| 3 | HC-SR04 Vcc — Arduino 5V |
| 4 | HC-SR04 Gnd — Arduino GND |
| 5 | HC-SR04 Trig — Arduino Digital Pin 7 |
| 6 | HC-SR04 Echo — Arduino Digital Pin 8 |
| 7 | Power the Arduino UNO (through the shield) from the PC using the USB cable |

Note: D10–D13 are taken by the shield (SPI), so do not wire anything else to them.

# Experiment 30 — 16x2 LCD (JHD162A) in 4-bit Mode

The LCD runs in 4-bit mode: RS→D12, E→D11, DB4–DB7→D5–D2. A 10 kΩ potentiometer sets the contrast on V0, the backlight is fed through 220 Ω, and R/W is tied to GND (write-only).

Figure 10: Circuit diagram of Experiment 30

## Connections

| # | Connection |
| --- | --- |
| 1 | Power the Arduino UNO from the PC using the USB cable |
| 2 | Arduino 5V — breadboard + rail; Arduino GND — breadboard − rail |
| 3 | LCD pin 1 (VSS) — − rail (GND) |
| 4 | LCD pin 2 (VDD) — + rail (5V) |
| 5 | Potentiometer outer leg A — + rail (5V) |
| 6 | Potentiometer outer leg B — − rail (GND) |
| 7 | Potentiometer wiper (middle leg) — LCD pin 3 (V0) (contrast) |
| 8 | LCD pin 4 (RS) — Arduino Digital Pin 12 |
| 9 | LCD pin 5 (R/W) — − rail (GND) (write-only) |
| 10 | LCD pin 6 (E) — Arduino Digital Pin 11 |
| 11 | LCD pins 7–10 (DB0–DB3) — not connected |
| 12 | LCD pin 11 (DB4) — Arduino Digital Pin 5 |
| 13 | LCD pin 12 (DB5) — Arduino Digital Pin 4 |
| 14 | LCD pin 13 (DB6) — Arduino Digital Pin 3 |
| 15 | LCD pin 14 (DB7) — Arduino Digital Pin 2 |
| 16 | LCD pin 15 (LED+) — one end of the 220 Ω resistor |
| 17 | 220 Ω resistor other end — + rail (5V) |
| 18 | LCD pin 16 (LED−) — − rail (GND) |

Note: In the diagram, every +5V tag connects to Arduino 5V and every GND tag to Arduino GND (use the breadboard rails). If you see only blocks or a blank screen, adjust the contrast pot first.

# Experiment 31 — Arduino B Reads Arduino A's Web Page

Board A (server, 192.168.1.177) runs the Experiment 18/19 web-server sketch. Board B (client, 192.168.1.178) connects to it on port 80, sends an HTTP GET and prints the returned page on its Serial Monitor.

Figure 11: Circuit diagram of Experiment 31

## Connections

| # | Connection (Board B) |
| --- | --- |
| 1 | Seat the Ethernet Shield onto Board B's UNO headers |
| 2 | Board B shield RJ45 port — network cable — the same router/switch Board A is connected to |
| 3 | Power Board B from the PC using the USB cable (Serial Monitor shows the received page) |

Note: Both boards must be on the same router or switch and must have different MAC addresses.