# Experiment Set 2 — Step-by-Step Lab Guide (Arduino UNO)

Submission due: 18/09/2026. This set moves from LEDs-only circuits into measurement (multimeter), analog sensing (potentiometer, LDR) and networking (Ethernet Shield). Wiring is still given as direct component-to-component connections — build each connection in order, landing both ends of a "connection" in the same breadboard row (or a power rail) to join them electrically. Nothing is skipped or referred back to "same as before" beyond what's explicitly noted; each experiment can be built starting from the previous one's hardware exactly as stated.

This guide tells you how to build and run each experiment. The graded report questions from the PDF are listed at the end of each experiment as a checklist — answer those yourself (they need your own explanation, photos/video, and calculations).

Experiments 18–20 (Ethernet Shield) reference sketches named `Experiment_18` / `Experiment_19` / `Experiment_20` that are supplied separately by the instructor (not printed in the slide deck — only their *behavior* and the questions about them are). The code shown for those three below is a standard, working Arduino Ethernet implementation that matches the described behavior and the PDF's own questions (11.b.1–4, library used, mac/IP setup, analog pins on a web page, ThingSpeak upload) — swap in the instructor's exact file if it differs, but the wiring and concepts are the same either way.

# 1. Components Used

Same physical kit as Experiment Set 1 (component-images/), plus three items that were listed but unused in Set 1 and are now active, plus one new item:

| Component | Photo | Pins / Legs |
| --- | --- | --- |
| Arduino UNO | component-images/arduino-uno.jpeg | Left header: 5V, RESET, 3.3V, 5V, GND, GND, VIN. Bottom header: A0–A5. Right headers: SCL, SDA, AREF, GND, 13, 12, 11~, 10~, 9~, 8 and 7, 6~, 5~, 4, 3~, 2, TX→1, RX←0. ~ = PWM-capable pin. |
| Breadboard | component-images/breadboard.jpeg | Top/bottom rails: red `+`, blue `−`. Center strips: rows A–E / F–J, columns 1–60; A–E not joined to F–J. |
| LED | component-images/led-light.jpeg | Long leg = anode (+), short leg = cathode (−). |
| Resistor | component-images/resistor.jpeg | 2 legs, no polarity. |
| Jumper wires | component-images/wire.jpeg | Male–male, for every connection below. |
| USB cable | component-images/arduino-connector.jpeg | USB-B end → Arduino, USB-A end → PC. |
| Potentiometer | component-images/potentiometer.jpeg | 3 legs: two outer legs are the fixed ends of the resistive track (connect across 5V/GND to form a divider), middle leg = wiper (variable voltage output — read this one). |
| LDR module / bare LDR | component-images/light-intensity-control-senor.jpeg | 2 legs, no polarity — a light-dependent resistor: resistance drops as light increases. |
| Digital multimeter | *(not in image set — any bench/handheld DMM)* | Set to the Ω (resistance) range for Experiments 11–13's resistor/LDR checks, and to DC Voltage (V⎓) range for the voltage measurements in Experiments 12–13. Red probe = +, black probe = COM/−. |
| Ethernet Shield (W5100-based) | *(not in image set)* | Stacks directly on top of the Arduino UNO — its header pins plug straight into the UNO's headers, pin-for-pin. Internally uses pins 10, 11, 12, 13 (SPI: SS/MOSI/MISO/SCK) to talk to the W5100 chip — those four pins are reserved and unavailable for LEDs while the shield is mounted. Has an RJ45 Ethernet port — connect it to your router/PC with a network cable for Experiments 18–20. |

# 2. One-Time Software Setup

| Step | From | To |
| --- | --- | --- |
| 1 | Arduino UNO USB port | USB cable (USB-B end) |
| 2 | USB cable (USB-A end) | PC USB port |
| 3 | IDE menu Tools → Board | Select Arduino AVR Boards → Arduino Uno |
| 4 | IDE menu Tools → Port | Select COM3 (Arduino UNO) *(use whichever COM number appears)* |
| 5 (Exp 18–20 only) | IDE menu Sketch → Include Library → Manage Libraries | Confirm Ethernet library is installed (bundled with the IDE by default) |
| 6 (Exp 18–20 only) | Ethernet Shield | Push firmly onto the UNO's headers so every pin row is seated, *then* plug in an Ethernet cable from the shield's RJ45 port to your router |
| 7 (Exp 20 only) | Browser | Create a free account at thingspeak.com (see Q20.d) before running Experiment 20 |

(Install the Arduino IDE first, from https://www.arduino.cc/, if not already installed.)

# 3. Breadboard / Measurement Layout Key

Every "connection" below is: plug both ends into the breadboard so they land in the same row (or the same power rail) — that's what makes them electrically joined.

Power rails: red strip = +, blue strip = −.

For Experiments 11–13, the Arduino only supplies 5V and GND — it is not running any sketch, so any USB connection (PC or a phone charger) that powers the board is enough.

When measuring voltage with the multimeter, probe *across* the two nodes you're measuring between — do not break the circuit to insert the meter. When measuring resistance, the component must be *isolated* from the powered circuit (multimeter pushes its own tiny test current through it).

# 4. Experiment-by-Experiment Build

## Experiment 11 — Review of Components (Multimeter)

No Arduino sketch — the board (powered over USB) is only used as a convenient 5V/GND source for Task 11.c; Tasks 11.a and 11.b use the multimeter standalone.

| # | Step |
| --- | --- |
| 1 | Set the multimeter dial to the Ω (resistance) range |
| 2 | Task 11.a: touch one probe to each leg of a resistor (off the breadboard, not powered) and read the value; repeat for at least 3 different resistors |
| 3 | Task 11.b: place the potentiometer's two outer legs across the meter probes *(or just probe outer-leg-to-wiper)* and read resistance; turn the knob to 3 different positions, recording the value at each |
| 4 | Task 11.c: probe across the LDR's two legs; take a reading, then change the light falling on it (cover it with your hand, shine a phone torch on it, etc.) and take at least 5 readings across a range of light levels |

Report checklist: 11.a (3 photos, resistor readings), 11.b (3 photos, pot readings at 3 settings), 11.c (5 photos + a graph of Light Intensity vs. LDR resistance + explanation of *why* resistance drops as light rises — photoconductivity: more incident photons free more charge carriers in the semiconductor), 11.d–11.h (from the LDR datasheet: material, power dissipation at 30 °C, operating temperature range, minimum dark resistance, peak spectral-response wavelength).

## Experiment 12 — Building a Potential Divider Circuit

| # | Connection |
| --- | --- |
| 1 | Power the Arduino UNO from the PC using the USB cable *(sketch not required — just used as a 5V/GND source)* |
| 2 | Resistor 1 — one end to Arduino 5V |
| 3 | Resistor 1 other end — Resistor 2 one end (same breadboard row) |
| 4 | Resistor 2 other end — Resistor 3 one end |
| 5 | Resistor 3 other end — Resistor 4 one end |
| 6 | Resistor 4 other end — Resistor 5 one end |
| 7 | Resistor 5 other end — Arduino GND |

This gives you 5 resistors in one series chain from 5V down to GND, with 4 accessible junction nodes between them (call them N1 after R1, N2 after R2, N3 after R3, N4 after R4).

Task 12.b — measuring: set the multimeter to DC Voltage, put the black probe on GND, and touch the red probe to each junction node in turn (and across individual resistors, node-to-node) to read several different combinations, e.g. V(5V→N1) across R1 alone, V(N1→GND) across R2+R3+R4+R5 together, etc.

Report checklist: 12.a (1 photo of the wired chain), 12.b (3 photos of voltage measurements across 3 different resistor combinations, each clearly showing the reading), 12.c (write the voltage-divider equation for one resistor in a 5-resistor series chain: V_Rn = V_total × (R_n / (R1+R2+R3+R4+R5))).

## Experiment 13 — LDR in a Potential Divider

| # | Connection |
| --- | --- |
| 1 | Power the Arduino UNO from the PC using the USB cable |
| 2 | LDR leg 1 — Arduino 5V |
| 3 | LDR leg 2 — one end of the resistor (same row = this is your measurement node) |
| 4 | Resistor other end — Arduino GND |

Task 13.b — measuring: multimeter on DC Voltage, black probe on GND, red probe on the LDR–resistor junction node. Take a reading, then vary the light on the LDR (shade it / torch it) and take readings at 3 different light intensities.

Report checklist: 13.a (1 photo of the wiring), 13.b (3 photos of voltage readings at 3 light levels), 13.c (write the divider equation for the node voltage: V_node = V_supply × R / (R_LDR + R) — note this is voltage across the *fixed resistor*; swap the ratio if you measured across the LDR instead).

## Experiment 14 — Blink Rate Follows Potentiometer Position

| # | Connection |
| --- | --- |
| 1 | Power the Arduino UNO from the PC using the USB cable |
| 2 | LED anode (long leg) — one end of the resistor |
| 3 | Resistor — other end to Arduino Digital Pin 13 |
| 4 | LED cathode (short leg) — Arduino GND |
| 5 | Potentiometer outer leg A — Arduino 5V |
| 6 | Potentiometer outer leg B — Arduino GND |
| 7 | Potentiometer wiper (middle leg) — Arduino A0 |

Code — sketch name `Experiment_14` (from the slide deck, verbatim):

int sensorPin = A0;   // select the input pin for the potentiometer
int ledPin = 13;      // select the pin for the LED
int sensorValue = 0;  // variable to store the value coming from the sensor

void setup() {
  pinMode(ledPin, OUTPUT);
}
void loop() {
  sensorValue = analogRead(sensorPin);
  digitalWrite(ledPin, HIGH);
  delay(sensorValue);   // 0–1023 ms, so blink rate slows as sensorValue rises
  digitalWrite(ledPin, LOW);
  delay(sensorValue);
}

Report checklist: 14.a (video with voice-over showing the LED's blink rate changing as you turn the pot), 14.b (explain analogRead: 10-bit ADC, returns 0–1023 for 0–5V), 14.c (draw the potentiometer's internal resistive-track + wiper diagram and explain how sliding the wiper changes the divider ratio), 14.d (ATmega328P 28-pin DIP diagram: mark A0–A5, all digital I/O pins, and all PWM ~ pins against their physical DIP pin numbers), 14.e (analog input is enabled by the chip's internal ADC / analog-to-digital converter multiplexed across A0–A5), 14.f (PWM pins are driven by the chip's Timer0/Timer1/Timer2 hardware timers in fast-PWM/phase-correct mode, toggling the pin automatically without CPU intervention each cycle).

## Experiment 15 — LED Intensity Follows Potentiometer Position

(Hardware from Experiment 14, with one change: the LED must move to a PWM pin since analogWrite needs one — pin 13 isn't PWM-capable.)

| # | Connection |
| --- | --- |
| 1 | Power the Arduino UNO from the PC using the USB cable |
| 2 | LED anode (long leg) — one end of the resistor |
| 3 | Resistor — other end to Arduino Digital Pin 9 *(marked 9~, PWM)* |
| 4 | LED cathode (short leg) — Arduino GND |
| 5 | Potentiometer outer leg A — Arduino 5V |
| 6 | Potentiometer outer leg B — Arduino GND |
| 7 | Potentiometer wiper (middle leg) — Arduino A0 |

Code:

int sensorPin = A0;
int ledPin = 9;         // moved to a PWM-capable pin
int sensorValue = 0;
int brightness = 0;

void setup() {
  pinMode(ledPin, OUTPUT);
}
void loop() {
  sensorValue = analogRead(sensorPin);           // 0–1023
  brightness = map(sensorValue, 0, 1023, 0, 255); // scale to analogWrite's 8-bit range
  analogWrite(ledPin, brightness);
}

Report checklist: 15.a (video with voice-over), 15.b (paste the new code), 15.c (describe the change: swapped delay(sensorValue) blinking on pin 13 for analogWrite brightness control on PWM pin 9, with map() rescaling the 10-bit ADC reading to the 8-bit PWM range).

## Experiment 16 — Blink Rate Follows Light Intensity (LDR replaces Potentiometer)

(Hardware from Experiment 14, with the potentiometer's divider swapped for an LDR divider — same idea as Experiment 13, but now tapped into A0 instead of just measured.)

| # | Connection |
| --- | --- |
| 1 | Power the Arduino UNO from the PC using the USB cable |
| 2 | LED anode (long leg) — one end of the resistor |
| 3 | Resistor — other end to Arduino Digital Pin 13 |
| 4 | LED cathode (short leg) — Arduino GND |
| 5 | LDR leg 1 — Arduino 5V |
| 6 | LDR leg 2 — one end of a second resistor (same row = this is your A0 tap node) |
| 7 | Second resistor's other end — Arduino GND |
| 8 | Same LDR/resistor junction node (step 6/7) — Arduino A0 |

Code: identical to Experiment 14 (Experiment_14 sketch, unchanged) — only the hardware feeding A0 changed from a potentiometer's wiper to an LDR divider's junction.

Report checklist: 16.a (video with voice-over showing blink rate change as you vary light on the LDR — cover it, shine a torch on it, etc.).

## Experiment 17 — LED Intensity Follows Light Intensity

(Hardware from Experiment 16, with the LED moved to PWM pin 9 exactly as in Experiment 15.)

| # | Connection |
| --- | --- |
| 1 | Power the Arduino UNO from the PC using the USB cable |
| 2 | LED anode (long leg) — one end of the resistor |
| 3 | Resistor — other end to Arduino Digital Pin 9 *(PWM)* |
| 4 | LED cathode (short leg) — Arduino GND |
| 5 | LDR leg 1 — Arduino 5V |
| 6 | LDR leg 2 — one end of a second resistor (A0 tap node) |
| 7 | Second resistor's other end — Arduino GND |
| 8 | Same LDR/resistor junction node — Arduino A0 |

Code: identical to Experiment 15 (analogRead(A0) → map() → analogWrite(9, brightness)) — only the sensor divider changed from potentiometer to LDR.

Report checklist: 17.a (video with voice-over), 17.b (paste the code — same as Exp 15), 17.c (describe: same brightness-control code as Experiment 15, input swapped from potentiometer wiper to LDR divider node, so brightness now tracks ambient light instead of knob position).

## Experiment 18 — Ethernet Shield: Web Page Showing Analog Inputs

| # | Connection |
| --- | --- |
| 1 | Seat the Ethernet Shield onto the Arduino UNO headers (all pins aligned, pressed flush) |
| 2 | Ethernet Shield RJ45 port — network cable — router/switch with internet/LAN access |
| 3 | Potentiometer outer leg A — Arduino 5V *(on the shield's passthrough header)* |
| 4 | Potentiometer outer leg B — Arduino GND |
| 5 | Potentiometer wiper — Arduino A0 |
| 6 | *(repeat a similar 3-leg divider for any other analog sensor you want on A1–A5, e.g. the LDR divider from Experiment 16/17 into A1)* |
| 7 | Power the Arduino UNO (through the shield) from the PC using the USB cable |

Code — sketch name `Experiment_18` (standard Arduino Ethernet "analog inputs on a web page" example — matches the PDF's Q18.b about library/mac/IP and Q18.d function list):

#include <SPI.h>
#include <Ethernet.h>

byte mac[] = { 0xDE, 0xAD, 0xBE, 0xEF, 0xFE, 0xED };  // any locally-administered MAC
IPAddress ip(192, 168, 1, 177);                        // pick a free address on your LAN

EthernetServer server(80);

void setup() {
  Serial.begin(9600);
  Ethernet.begin(mac, ip);
  Serial.print("Hardware status: ");
  Serial.println(Ethernet.hardwareStatus());
  Serial.print("Link status: ");
  Serial.println(Ethernet.linkStatus());
  server.begin();
  Serial.print("Server is at ");
  Serial.println(Ethernet.localIP());
}

void loop() {
  EthernetClient client = server.available();
  if (client) {
    boolean blankLine = true;
    while (client.connected()) {
      if (client.available()) {
        char c = client.read();
        if (c == '\n' && blankLine) {
          client.println("HTTP/1.1 200 OK");
          client.println("Content-Type: text/html");
          client.println("Connection: close");
          client.println();
          client.println("<!DOCTYPE HTML><html>");
          for (int a = 0; a < 6; a++) {
            client.print("analog input ");
            client.print(a);
            client.print(" is ");
            client.print(analogRead(a));
            client.println("<br />");
          }
          client.println("</html>");
          break;
        }
        blankLine = (c == '\n') ? true : (c == '\r') ? blankLine : false;
      }
    }
    delay(1);
    client.stop();
  }
}

Report checklist: 18.a (video with voice-over showing you browse to the board's IP and see the page update), 18.b (18.b.1: SPI.h + Ethernet.h; 18.b.2: byte mac[] = {...} array passed to Ethernet.begin; 18.b.3: an IPAddress object passed the same call, or left out entirely for DHCP; 18.b.4: data goes out as raw HTTP text over TCP port 80 via EthernetClient/EthernetServer), 18.c (shield uses UNO pins 10 (SS), 11 (MOSI), 12 (MISO), 13 (SCK) for SPI to the W5100 — map these to their ATmega328P DIP pins and explain the chip's hardware SPI peripheral enables this), 18.d (explain each listed function: Serial.begin opens the serial port at a baud rate; Ethernet.begin configures the network interface; hardwareStatus()/linkStatus() report shield presence/cable-link state; server.begin() starts listening; EthernetServer server(80) constructs a server object bound to HTTP port 80; Serial.print/println write to the USB serial monitor; client.print/println write bytes back to the connected browser).

## Experiment 19 — Web Page Shows Only Two Labelled Sensors

(Hardware builds on Experiment 18 — keep the potentiometer on A0, add the LDR divider from Experiment 16/17 wired into A1, and move the potentiometer's wiper tap from A0 to A4.)

| # | Connection |
| --- | --- |
| 1–2 | Ethernet Shield seated + networked, as in Experiment 18 |
| 3 | LDR leg 1 — Arduino 5V |
| 4 | LDR leg 2 — one end of a resistor (junction = A1 tap) |
| 5 | Resistor other end — Arduino GND |
| 6 | LDR/resistor junction (step 4/5) — Arduino A1 |
| 7 | Potentiometer outer leg A — Arduino 5V |
| 8 | Potentiometer outer leg B — Arduino GND |
| 9 | Potentiometer wiper — Arduino A4 |

Code: same as Experiment 18, with the reporting loop replaced:

client.println("<!DOCTYPE HTML><html>");
          client.print("The current LDR value is ");
          client.print(analogRead(A1));
          client.println("<br />");
          client.print("The current Potentiometer value is ");
          client.print(analogRead(A4));
          client.println("<br />");
          client.println("</html>");

Report checklist: 19.a (video with voice-over — change light on the LDR and turn the pot, show both labelled values update on the page), 19.b (paste the full new sketch), 19.c (describe the change: restricted the for loop over all 6 analog pins down to two explicit, labelled reads at A1 and A4).

## Experiment 20 — Potentiometer to Serial Monitor *and* ThingSpeak

(Hardware builds on Experiment 19 — this experiment only needs the potentiometer on A0; the LDR/A1 wiring from Experiment 19 can stay in place but isn't required.)

| # | Connection |
| --- | --- |
| 1–2 | Ethernet Shield seated + networked, as in Experiment 18 |
| 3 | Potentiometer outer leg A — Arduino 5V |
| 4 | Potentiometer outer leg B — Arduino GND |
| 5 | Potentiometer wiper — Arduino A0 |

Code — sketch name `Experiment_20` (adds a ThingSpeak upload alongside the existing Serial + local web page output):

#include <SPI.h>
#include <Ethernet.h>

byte mac[] = { 0xDE, 0xAD, 0xBE, 0xEF, 0xFE, 0xED };
EthernetClient client;
char server[] = "api.thingspeak.com";
String writeAPIKey = "YOUR_THINGSPEAK_WRITE_API_KEY";  // from your channel's API Keys tab
unsigned long lastUpdate = 0;

void setup() {
  Serial.begin(9600);
  Ethernet.begin(mac);   // DHCP; use Ethernet.begin(mac, ip) if your router has none
}

void loop() {
  int potValue = analogRead(A0);
  Serial.print("Potentiometer value: ");
  Serial.println(potValue);

  if (millis() - lastUpdate > 15000) {   // ThingSpeak free tier: 1 update / 15 s minimum
    lastUpdate = millis();
    if (client.connect(server, 80)) {
      String body = "api_key=" + writeAPIKey + "&field1=" + String(potValue);
      client.println("POST /update HTTP/1.1");
      client.println("Host: api.thingspeak.com");
      client.println("Connection: close");
      client.println("Content-Type: application/x-www-form-urlencoded");
      client.print("Content-Length: ");
      client.println(body.length());
      client.println();
      client.println(body);
      client.stop();
    }
  }
  delay(1000);
}

Report checklist: 20.a (video with voice-over — turn the pot, show the Serial Monitor value change *and* the ThingSpeak channel graph update), 20.b (paste the sketch), 20.c (describe changes vs. Exp 19: dropped the local web server entirely, added an EthernetClient that POSTs the pot reading to ThingSpeak's /update endpoint every 15 s, plus a Serial.println for local viewing), 20.d (ThingSpeak account steps: sign up at thingspeak.com → create a new Channel → note Channel ID → open its API Keys tab → copy the Write API Key), 20.e (how data reaches ThingSpeak: the sketch opens a TCP connection to api.thingspeak.com:80 and sends an HTTP POST to /update with api_key and field1=<value> in the request body — ThingSpeak logs each POST as one datapoint on Field 1's chart), 20.f (how data reaches the Serial Monitor: Serial.println(potValue) writes the ADC reading to the USB serial connection each loop, viewable via IDE Tools → Serial Monitor at 9600 baud).

# 5. Quick Reference Summary

| Experiment | Analog pin(s) | Digital / PWM pin(s) | Extra hardware beyond Set 1's LED+resistor |
| --- | --- | --- | --- |
| 11 | — | — | Multimeter only (resistors, potentiometer, LDR) |
| 12 | — | — | 5 resistors in series + multimeter (voltage) |
| 13 | — | — | LDR + resistor divider + multimeter (voltage) |
| 14 | A0 (pot) | 13 | Potentiometer |
| 15 | A0 (pot) | 9 (PWM) | Potentiometer |
| 16 | A0 (LDR) | 13 | LDR + resistor divider |
| 17 | A0 (LDR) | 9 (PWM) | LDR + resistor divider |
| 18 | A0–A5 (any sensors) | 10/11/12/13 (SPI, reserved by shield) | Ethernet Shield |
| 19 | A1 (LDR) + A4 (pot) | 10/11/12/13 (SPI, reserved) | Ethernet Shield + LDR divider + potentiometer |
| 20 | A0 (pot) | 10/11/12/13 (SPI, reserved) | Ethernet Shield + ThingSpeak account |