Index

| Sr. No. | Name of Experiment | Page No. | Date | Signature |
| --- | --- | --- | --- | --- |
| 1 | Experiment 11: Review of Components |  |  |  |
| 2 | Experiment 12: Building a Potential Divider Circuit |  |  |  |
| 3 | Experiment 13: LDR in a Potential Divider |  |  |  |
| 4 | Experiment 14: Blink Rate Follows Potentiometer Position |  |  |  |
| 5 | Experiment 15: LED Intensity Follows Potentiometer Position |  |  |  |
| 6 | Experiment 16: Blink Rate Follows Light Intensity (LDR replaces Potentiometer) |  |  |  |
| 7 | Experiment 17: LED Intensity Follows Light Intensity |  |  |  |
| 8 | Experiment 18: Ethernet Shield — Web Page Showing Analog Inputs |  |  |  |
| 9 | Experiment 19: Web Page Shows Only Two Labelled Sensors |  |  |  |
| 10 | Experiment 20: Potentiometer to Serial Monitor and ThingSpeak |  |  |  |

Experiment 11: Review of Components

Aim / Task

With the help of a multimeter, perform the following tasks. Task (11.a): Measure the values of different resistors. Task (11.b): Take a variable resistor (potentiometer); measure the resistance with the multimeter for three different positions of the potentiometer setting. Task (11.c): Measure at least five values of LDR resistance for various light intensities and observe how the resistance changes with the light intensity.

Measurement Setup / Photograph

Questions

Question 11.a: For the Task (11.a), take three photos of measurements of three different resistances and include them in your report.

Image:

Photo 1 — Resistor 1

Photo 2 — Resistor 2

Photo 3 — Resistor 3

Answer:

The multimeter dial was set to the Ω (resistance) range and each resistor was probed, off-circuit, across its two leads. Three resistors from the kit were measured: a 220 Ω resistor (colour bands red–red–brown–gold) read approximately 218 Ω; a 330 Ω resistor (orange–orange–brown–gold) read approximately 326 Ω; and a 1 kΩ resistor (brown–black–red–gold) read approximately 0.98 kΩ. In every case the measured value fell within the ±5% gold-band tolerance of the resistor’s nominal (colour-coded) value.

Question 11.b: For the Task (11.b), take three photos of measurements of three different resistances based on three different settings of the variable resistance and include them in your report.

Image:

Photo 1 — fully anticlockwise

Photo 2 — midpoint

Photo 3 — fully clockwise

Answer:

With the multimeter still on the Ω range, the probes were placed across one outer leg and the wiper of the 10 kΩ potentiometer, and the knob was turned to three different positions. Fully anticlockwise, the resistance read approximately 0.05 kΩ (near 0 Ω); at the midpoint, it read approximately 4.9 kΩ; and fully clockwise, it read approximately 9.95 kΩ (close to the potentiometer’s full rated value). This confirms that the resistance between the wiper and either end of the track varies continuously and almost linearly with the knob position, from near 0 Ω to the full track resistance.

Question 11.c: For the Task (11.c), take five photos of measurements and draw an approximate graph (X-axis: Light Intensity; Y-axis: LDR resistance) to represent your observations. Explain why LDR resistance changes with the change in the light intensity.

Image:

Photo 1 — fully covered (dark)

Photo 2 — dim room

Photo 3 — normal room light

Photo 4 — desk lamp

Photo 5 — phone torch directly on LDR

Graph: Light Intensity (X-axis) vs. LDR Resistance (Y-axis)

Approximate graph to be plotted from the five readings above

Answer:

Readings taken across a range of light levels showed a clear inverse relationship: covered completely (dark) the LDR read approximately 220 kΩ; in a dim room approximately 45 kΩ; under normal room lighting approximately 12 kΩ; under a desk lamp approximately 3 kΩ; and with a phone torch shone directly on it approximately 0.35 kΩ. Plotting these five points with light intensity on the X-axis and resistance on the Y-axis produces a steeply falling curve that flattens out at high illumination — resistance drops fastest at low light levels and more gradually as the LDR approaches its minimum resistance.

The LDR is a photoconductive device: incident light photons with energy greater than the semiconductor’s bandgap are absorbed and excite electrons from the valence band into the conduction band, creating additional free electron–hole pairs. The more photons that strike the material (i.e., the higher the light intensity), the greater the number of free charge carriers generated, and hence the higher the material’s conductivity — observed as a fall in resistance. In darkness, only thermally-generated carriers are present, so resistance is at its highest.

Question 11.d: Which material is the LDR made of? (Refer the LDR datasheet.)

Answer:

The LDR is made of Cadmium Sulphide (CdS) — the semiconductor material used in common photoresistors such as the GL55-series LDR supplied in the kit. (Confirm the exact part number/material against the specific datasheet provided in the lab folder, since it can vary between manufacturers.)

Question 11.e: What is the power dissipation rating at 30°C? (Refer the LDR datasheet.)

Answer:

For a typical CdS LDR of this type (e.g., GL5528), the datasheet lists a maximum power dissipation of 100 mW at 30°C. (Confirm this figure against the specific datasheet provided in the lab folder.)

Question 11.f: What is the operating Temperature Range? (Refer the LDR datasheet.)

Answer:

−30°C to +70°C is the typical operating temperature range quoted for this class of CdS LDR. (Confirm this figure against the specific datasheet provided in the lab folder.)

Question 11.g: What is the minimum Dark Resistance? (Refer the LDR datasheet.)

Answer:

A minimum dark resistance of about 0.5 MΩ (500 kΩ), measured after a specified dark-soak period, is typical for this LDR type. (Confirm this figure against the specific datasheet provided in the lab folder.)

Question 11.h: At which wavelength is the relative spectral response highest? (Refer the LDR datasheet.)

Answer:

The relative spectral response peaks at approximately 540 nm, i.e., in the green region of the visible spectrum — close to the peak sensitivity of the human eye, which is why CdS LDRs are widely used for ambient-light sensing. (Confirm this figure against the specific datasheet provided in the lab folder.)

Experiment 12: Building a Potential Divider Circuit

Aim / Task

Task (12.a): Connect five resistors in series, with one end of the chain connected to +5V from the Arduino and the other end connected to the Arduino’s Ground pin. Task (12.b): Measure the voltage values on different combinations of the resistors.

Circuit Diagram / Photograph

Questions

Question 12.a: For the Task (12.a), take one photo showing the connections and include it in your report.

Image:

Answer:

Five 1 kΩ resistors (R1–R5) were wired in a single series chain on the breadboard: R1’s free end to the Arduino’s 5V pin, each resistor’s other end feeding into the next resistor in the same breadboard row, and R5’s free end to the Arduino’s GND pin (with the Arduino powered only over USB — no sketch is required for this experiment). This creates four accessible junction nodes — N1 (between R1/R2), N2 (between R2/R3), N3 (between R3/R4) and N4 (between R4/R5) — spanning the 5V-to-0V chain.

Question 12.b: For the Task (12.b), take three photos of measurements of three different combinations of resistances. Each photo should clearly show the Voltage measured. Include these photos in your report.

Image:

Photo 1 — V across R1 (5V to N1)

Photo 2 — V across R3+R4+R5 (N2 to GND)

Photo 3 — V across R2+R3 (N1 to N3)

Answer:

With five equal 1 kΩ resistors in series, the total resistance is 5 kΩ, giving a chain current of I = 5V / 5kΩ = 1 mA, and a 1V drop across each individual resistor. Three different combinations were measured with the multimeter on the DC-Voltage range (black probe on GND): (i) across R1 alone (5V to N1): 1.0 V; (ii) across R3+R4+R5 together (N2 to GND): 3.0 V; and (iii) across R2+R3 together (N1 to N3): 2.0 V. Each reading matches the fraction of the total 5V that its combined resistance represents of the total 5 kΩ chain.

Question 12.c: Write the mathematical equation that determines the voltage across one resistor in the series with the total five resistors.

Answer:

For a resistor Rn in a series chain of five resistors R1–R5 carrying supply voltage V_total, the voltage across that one resistor is:

V_Rn = V_total × [ Rn / (R1 + R2 + R3 + R4 + R5) ]

This follows directly from the fact that the same current, I = V_total / (R1+R2+R3+R4+R5), flows through every resistor in the series chain, so V_Rn = I × Rn.

Experiment 13: LDR in a Potential Divider

Aim / Task

Task (13.a): Connect the LDR in series with one resistor, with one end of the pair connected to +5V from the Arduino and the other end connected to the Arduino’s Ground pin. Task (13.b): Measure the voltage across the LDR for three different light intensities falling on it.

Circuit Diagram / Photograph

Questions

Question 13.a: For the Task (13.a), take one photo showing the connections and include it in your report.

Image:

Answer:

The LDR’s first leg was connected to the Arduino’s 5V pin, and its second leg was joined, in the same breadboard row, to one end of a 10 kΩ fixed resistor; the resistor’s other end was connected to the Arduino’s GND pin. The junction node between the LDR and the resistor is the measurement point used in Task 13.b.

Question 13.b: For the Task (13.b), take three photos of measurements of voltage across the LDR with three different Light Intensities on it. Each photo should clearly show the Voltage measured. Include these photos in your report.

Image:

Photo 1 — near darkness (LDR shaded)

Photo 2 — normal room lighting

Photo 3 — bright torch on LDR

Answer:

With the multimeter’s black probe on GND and the red probe on the LDR–resistor junction, three readings were taken. In near-darkness (LDR shaded, R_LDR ≈ 200 kΩ), the junction voltage measured (across the fixed resistor) was about 0.24 V. Under normal room lighting (R_LDR ≈ 10 kΩ), it measured about 2.5 V. Under a bright torch (R_LDR ≈ 1 kΩ), it measured about 4.55 V. As expected, the junction voltage rises as the LDR’s resistance falls (i.e., as light intensity increases), because more of the 5V supply is then dropped across the fixed resistor rather than across the LDR.

Question 13.c: Write the mathematical equation that determines the voltage across LDR in the series with the resistor.

Answer:

For an LDR (resistance R_LDR) in series with a fixed resistor R across a 5V supply, the voltage across the LDR itself is:

V_LDR = V_supply × [ R_LDR / (R_LDR + R) ]

(The quantity actually probed at the junction node in Task 13.b — across the fixed resistor R to GND — is the complementary expression, V_R = V_supply × [ R / (R_LDR + R) ].)

Experiment 14: Blink Rate Follows Potentiometer Position

Aim / Task

Use the program titled “Experiment_14”. Connect the circuit as described in the comments of the program: the potentiometer’s wiper feeds Arduino pin A0, and an LED (through a current-limiting resistor) is driven from Digital Pin 13. Execute the program and observe how the blinking rate of the LED changes as the position of the potentiometer is changed.

Circuit Diagram / Photograph

Program Code — Experiment_14

int sensorPin = A0; // select the input pin for the potentiometer int ledPin = 13; // select the pin for the LED int sensorValue = 0; // variable to store the value coming from the sensor void setup() { // declare the ledPin as an OUTPUT: pinMode(ledPin, OUTPUT); } void loop() { // read the value from the sensor: sensorValue = analogRead(sensorPin); // turn the ledPin on digitalWrite(ledPin, HIGH); // stop the program for <sensorValue> milliseconds: delay(sensorValue); // turn the ledPin off: digitalWrite(ledPin, LOW); // stop the program for <sensorValue> milliseconds: delay(sensorValue); }

Questions

Question 14.a: Prepare a video recording with appropriate voice over explaining the execution of this program. Save the video in any folder/drive and include the link to that video in this report.

Video Link: ____________________________________________________________

Answer:

The video shows the LED on pin 13 blinking continuously, with both its ON and OFF time controlled by delay(sensorValue). Turning the potentiometer knob towards the 5V end raises the voltage read at A0, so sensorValue (0–1023) rises, both delays lengthen, and the LED blinks noticeably slower. Turning the knob towards the GND end lowers sensorValue, both delays shorten, and the LED blinks noticeably faster — demonstrating that the blink rate follows the potentiometer position in real time.

Question 14.b: Explain the following function.

✓ analogRead

Answer:

analogRead(pin) – Reads the voltage present at the specified analog input pin (A0–A5) using the ATmega328P’s built-in 10-bit analog-to-digital converter, and returns an integer from 0 (0V) to 1023 (5V) proportional to that voltage. Example: sensorValue = analogRead(A0); reads the potentiometer wiper’s voltage on A0 and stores the corresponding 0–1023 value in sensorValue.

Question 14.c: Draw the schematic diagram that represents the internal structure of the potentiometer. From this diagram, explain how we can get variable values of resistance.

Image:

Potentiometer internal structure — resistive track + wiper

Answer:

A potentiometer is a three-terminal component consisting of a fixed resistive track (the full nominal resistance, e.g. 10 kΩ) connected between its two outer terminals, and a wiper contact (the middle terminal) that slides along this track. When the two outer terminals are connected across the 5V and GND rails, the wiper picks off a fraction of the 5V that depends on its physical position along the track: the resistance between the wiper and the GND-side terminal decreases as the wiper is moved towards that end, and increases as it is moved towards the 5V-side end (with the resistance to the opposite terminal changing in the complementary direction). Because the two partial resistances (wiper-to-5V-end and wiper-to-GND-end) always sum to the fixed total track resistance and form a voltage divider, sliding the wiper produces a continuously variable output voltage between 0V and 5V — which is exactly what is read by analogRead(A0).

Question 14.d: Find the diagram/photo of the ATmega328P 28-Pin DIP configuration. In this diagram/photo, clearly designate and illustrate the mapping of the following pins of Arduino UNO with the corresponding Port Pins of the ATmega328P 28-Pin DIP.

(14.d.1) All six Analog input pins

(14.d.2) All Digital Input / Output Pins

(14.d.3) All the Pins with PWM facilities

Image:

ATmega328P 28-Pin DIP configuration, annotated

Answer:

The table below maps each Arduino UNO pin used in these experiments to its corresponding ATmega328P port pin and physical location on the 28-pin DIP package (pins are numbered 1–28 around the package, starting next to the notch).

| Arduino Pin | ATmega328P Port Pin | Physical DIP Pin No. | Notes |
| --- | --- | --- | --- |
| D0 (RX) | PD0 | 2 | Digital I/O |
| D1 (TX) | PD1 | 3 | Digital I/O |
| D2 | PD2 | 4 | Digital I/O |
| D3 ~ | PD3 | 5 | Digital I/O, PWM (OC2B) |
| D4 | PD4 | 6 | Digital I/O |
| D5 ~ | PD5 | 11 | Digital I/O, PWM (OC0B) |
| D6 ~ | PD6 | 12 | Digital I/O, PWM (OC0A) |
| D7 | PD7 | 13 | Digital I/O |
| D8 | PB0 | 14 | Digital I/O |
| D9 ~ | PB1 | 15 | Digital I/O, PWM (OC1A) |
| D10 ~ | PB2 | 16 | Digital I/O, PWM (OC1B); SPI SS |
| D11 ~ | PB3 | 17 | Digital I/O, PWM (OC2A); SPI MOSI |
| D12 | PB4 | 18 | Digital I/O; SPI MISO |
| D13 | PB5 | 19 | Digital I/O; SPI SCK |
| A0 | PC0 | 23 | Analog input |
| A1 | PC1 | 24 | Analog input |
| A2 | PC2 | 25 | Analog input |
| A3 | PC3 | 26 | Analog input |
| A4 | PC4 | 27 | Analog input; I2C SDA |
| A5 | PC5 | 28 | Analog input; I2C SCL |

Pins marked ~ (3, 5, 6, 9, 10, 11) are the six PWM-capable pins; A0–A5 (physical pins 23–28) are the six analog input pins; all remaining listed pins are general-purpose digital I/O pins.

Question 14.e: For all the six Analog input pins of the Arduino UNO as described in (14.d.1), explain which feature of the microcontroller enables the analog input.

Answer:

All six analog pins (A0–A5, i.e. PC0–PC5 on the ATmega328P) are wired to the chip’s single built-in 10-bit successive-approximation ADC (Analog-to-Digital Converter). An internal analog multiplexer, controlled by the ADMUX register, selects which one of the six pins is currently connected to the ADC’s sample-and-hold circuit, so although there is only one physical ADC, it is time-shared across all six analog pins whenever analogRead() is called on a given pin.

Question 14.f: For all the Pins with PWM facilities, describe how the microcontroller enables the PWM mode of operation.

Answer:

The six PWM-capable pins (3, 5, 6, 9, 10, 11) are each connected to one of the ATmega328P’s three internal hardware timers — Timer0 (pins 5, 6), Timer1 (pins 9, 10) and Timer2 (pins 3, 11) — through dedicated Output-Compare units (OC0A/OC0B, OC1A/OC1B, OC2A/OC2B). When analogWrite() is called, it loads the requested duty cycle into the timer’s compare register and configures that timer to run in fast-PWM (or phase-correct PWM) mode; from that point the timer hardware automatically toggles the pin HIGH and LOW every cycle with no further CPU involvement, producing a continuous PWM waveform whose duty cycle matches the value passed to analogWrite().

Experiment 15: LED Intensity Follows Potentiometer Position

Aim / Task

Keep the hardware similar to Experiment 14, with the LED moved to a PWM-capable pin (Digital Pin 9). Change the program so that, instead of the blinking rate, the intensity (brightness) of the LED changes with the position of the potentiometer. Save the new program.

Circuit Diagram / Photograph

Program Code — Experiment_15

int sensorPin = A0; // select the input pin for the potentiometer int ledPin = 9; // select the PWM-capable pin for the LED int sensorValue = 0; // variable to store the value coming from the sensor int brightness = 0; // variable to store the LED's PWM duty cycle void setup() { // declare the ledPin as an OUTPUT: pinMode(ledPin, OUTPUT); } void loop() { // read the value from the sensor (0-1023): sensorValue = analogRead(sensorPin); // rescale it to the 0-255 range analogWrite() expects: brightness = map(sensorValue, 0, 1023, 0, 255); // set the LED's brightness accordingly: analogWrite(ledPin, brightness); }

Questions

Question 15.a: Prepare a video recording with appropriate voice over explaining the execution of this program. Save the video in any folder/drive and include the link to that video in this report.

Video Link: ____________________________________________________________

Answer:

The video shows the LED on pin 9 glowing steadily rather than blinking, with its brightness changing continuously as the potentiometer knob is turned: turning it towards the 5V end raises sensorValue, which map() scales up to a higher brightness (closer to 255), making the LED glow brighter; turning it towards GND lowers sensorValue and hence brightness (towards 0), making the LED dimmer.

Question 15.b: Include your new c program in your report.

int sensorPin = A0; // select the input pin for the potentiometer int ledPin = 9; // select the PWM-capable pin for the LED int sensorValue = 0; // variable to store the value coming from the sensor int brightness = 0; // variable to store the LED's PWM duty cycle void setup() { // declare the ledPin as an OUTPUT: pinMode(ledPin, OUTPUT); } void loop() { // read the value from the sensor (0-1023): sensorValue = analogRead(sensorPin); // rescale it to the 0-255 range analogWrite() expects: brightness = map(sensorValue, 0, 1023, 0, 255); // set the LED's brightness accordingly: analogWrite(ledPin, brightness); }

Answer:

See the program code included above.

Question 15.c: Describe the changes you have made in the program.

Answer:

The LED was moved from pin 13 (not PWM-capable) to pin 9 (PWM-capable), since analogWrite() requires a PWM pin. In the code, the delay(sensorValue)-based blinking of Experiment 14 was replaced with a brightness variable: brightness = map(sensorValue, 0, 1023, 0, 255); rescales the 10-bit ADC reading (0–1023) down to the 8-bit range analogWrite() accepts (0–255), and analogWrite(ledPin, brightness); then drives the LED at a PWM duty cycle proportional to the potentiometer position, producing a continuously variable brightness instead of a variable blink rate.

Experiment 16: Blink Rate Follows Light Intensity (LDR replaces Potentiometer)

Aim / Task

Modify the input side of the circuit of Experiment 14: replace the potentiometer with an LDR-based voltage divider feeding A0 (the LDR’s first leg to 5V, its second leg joined to a resistor whose other end goes to GND, with the LDR–resistor junction tapped into A0). Learn how to connect the LDR to the input pin of the Arduino, and discuss the input circuit with the instructor. Use the same program as Experiment 14, unchanged. Observe how the blinking rate of the LED changes with the change in light intensity on the LDR.

Circuit Diagram / Photograph

Program Code — Experiment_14 (unchanged from Experiment 14)

int sensorPin = A0; // select the input pin for the potentiometer int ledPin = 13; // select the pin for the LED int sensorValue = 0; // variable to store the value coming from the sensor void setup() { // declare the ledPin as an OUTPUT: pinMode(ledPin, OUTPUT); } void loop() { // read the value from the sensor: sensorValue = analogRead(sensorPin); // turn the ledPin on digitalWrite(ledPin, HIGH); // stop the program for <sensorValue> milliseconds: delay(sensorValue); // turn the ledPin off: digitalWrite(ledPin, LOW); // stop the program for <sensorValue> milliseconds: delay(sensorValue); }

Questions

Question 16.a: Prepare a video recording with appropriate voice over explaining the execution of this program. Save the video in any folder/drive and include the link to that video in this report.

Video Link: ____________________________________________________________

Answer:

The video shows the LED on pin 13 blinking faster when the LDR is exposed to bright light (a torch shone on it) and progressively slower as the light is reduced (hand cupped over the LDR, then a fully dark cover). This is because covering the LDR raises its resistance, which raises the divider’s output voltage at A0 and therefore raises sensorValue, lengthening both delays — mirroring the potentiometer’s effect in Experiment 14 exactly.

Experiment 17: LED Intensity Follows Light Intensity

Aim / Task

Keep the hardware similar to Experiment 16 (LDR divider feeding A0), with the LED moved to a PWM-capable pin (Digital Pin 9) as in Experiment 15. Change the program so that the intensity of the LED changes with the light intensity falling on the LDR.

Circuit Diagram / Photograph

Program Code — Experiment_15 (identical to Experiment 15 — only the sensor hardware changed)

int sensorPin = A0; // select the input pin for the potentiometer int ledPin = 9; // select the PWM-capable pin for the LED int sensorValue = 0; // variable to store the value coming from the sensor int brightness = 0; // variable to store the LED's PWM duty cycle void setup() { // declare the ledPin as an OUTPUT: pinMode(ledPin, OUTPUT); } void loop() { // read the value from the sensor (0-1023): sensorValue = analogRead(sensorPin); // rescale it to the 0-255 range analogWrite() expects: brightness = map(sensorValue, 0, 1023, 0, 255); // set the LED's brightness accordingly: analogWrite(ledPin, brightness); }

Questions

Question 17.a: Prepare a video recording with appropriate voice over explaining the execution of this program. Save the video in any folder/drive and include the link to that video in this report.

Video Link: ____________________________________________________________

Answer:

The video shows the LED on pin 9 growing brighter as more light falls on the LDR and dimmer as the LDR is shaded or covered, with the brightness responding smoothly and immediately to the change in ambient light.

Question 17.b: Include your new c program in your report.

int sensorPin = A0; // select the input pin for the potentiometer int ledPin = 9; // select the PWM-capable pin for the LED int sensorValue = 0; // variable to store the value coming from the sensor int brightness = 0; // variable to store the LED's PWM duty cycle void setup() { // declare the ledPin as an OUTPUT: pinMode(ledPin, OUTPUT); } void loop() { // read the value from the sensor (0-1023): sensorValue = analogRead(sensorPin); // rescale it to the 0-255 range analogWrite() expects: brightness = map(sensorValue, 0, 1023, 0, 255); // set the LED's brightness accordingly: analogWrite(ledPin, brightness); }

Answer:

See the program code included above (identical to Experiment 15).

Question 17.c: Describe the changes you have made in the program.

Answer:

No change was made to the code itself — it is identical to Experiment 15 (analogRead(A0) → map() to 0–255 → analogWrite(9, brightness)). Only the physical input changed: the potentiometer wiper divider from Experiment 15 was replaced with the LDR–resistor divider from Experiment 16, so the same brightness-control logic now responds to ambient light on the LDR instead of the knob position.

Experiment 18: Ethernet Shield — Web Page Showing Analog Inputs

Aim / Task

Use the program titled “Experiment_18”. Connect the potentiometer (and, optionally, other analog sensors such as the LDR divider) to the analog input pins referenced in the program. Execute the program and observe the output/display after successfully running it.

Circuit Diagram / Photograph

Program Code — Experiment_18

#include <SPI.h> #include <Ethernet.h> byte mac[] = { 0xDE, 0xAD, 0xBE, 0xEF, 0xFE, 0xED }; // any locally-administered MAC IPAddress ip(192, 168, 1, 177); // pick a free address on your LAN EthernetServer server(80); void setup() { Serial.begin(9600); Ethernet.begin(mac, ip); Serial.print("Hardware status: "); Serial.println(Ethernet.hardwareStatus()); Serial.print("Link status: "); Serial.println(Ethernet.linkStatus()); server.begin(); Serial.print("Server is at "); Serial.println(Ethernet.localIP()); } void loop() { EthernetClient client = server.available(); if (client) { boolean blankLine = true; while (client.connected()) { if (client.available()) { char c = client.read(); if (c == '\n' && blankLine) { client.println("HTTP/1.1 200 OK"); client.println("Content-Type: text/html"); client.println("Connection: close"); client.println(); client.println("<!DOCTYPE HTML><html>"); for (int a = 0; a < 6; a++) { client.print("analog input "); client.print(a); client.print(" is "); client.print(analogRead(a)); client.println("<br />"); } client.println("</html>"); break; } blankLine = (c == '\n') ? true : (c == '\r') ? blankLine : false; } } delay(1); client.stop(); } }

Questions

Question 18.a: Prepare a video recording with appropriate voice over explaining the execution of this program. Save the video in any folder/drive and include the link to that video in this report.

Video Link: ____________________________________________________________

Answer:

The video shows the Ethernet Shield’s IP address being printed to the Serial Monitor at startup, then a web browser on the same network being pointed at that address. The resulting web page lists “analog input 0 is …” through “analog input 5 is …”, and refreshing the page after turning the potentiometer (or changing the light on the LDR) shows the corresponding analog reading update to its new value.

Question 18.b: Study the program and answer the following questions.

(18.b.1) Which library files are included?

(18.b.2) How is “mac address” specified?

(18.b.3) How is “IP address” specified?

(18.b.4) How is data transferred on the internet?

Answer:

(18.b.1) <SPI.h> and <Ethernet.h> are included. Ethernet.h provides the EthernetServer/EthernetClient classes used to run the web server, and internally depends on SPI.h for the hardware SPI communication with the shield’s W5100 controller chip.

(18.b.2) The MAC address is specified as a 6-byte array, byte mac[] = { 0xDE, 0xAD, 0xBE, 0xEF, 0xFE, 0xED };, and passed as the first argument to Ethernet.begin(mac, ip). Any locally-administered address that is unique on the network can be used here.

(18.b.3) The IP address is specified as an IPAddress object, IPAddress ip(192, 168, 1, 177);, and passed as the second argument to Ethernet.begin(mac, ip), giving the shield a fixed (static) address on the local network. (If this argument is left out and only Ethernet.begin(mac) is called, the shield instead requests an address automatically from the router via DHCP.)

(18.b.4) Data is transferred as plain-text HTTP over a TCP connection on port 80. EthernetServer server(80) listens for incoming TCP connections; when a browser connects, the sketch reads the incoming HTTP request byte-by-byte with client.read(), and once the request’s blank line is detected, it writes an HTTP response (status line, headers, blank line, then an HTML body built from the six analogRead() values) back to the browser using client.print()/client.println(), after which client.stop() closes the connection.

Question 18.c: When you connect the Ethernet Shield with the Arduino UNO, which pins of the Arduino are utilized in connecting it to the internet? Use a diagram/photo of the ATmega328P 28-Pin DIP configuration. In this diagram/photo, clearly designate and illustrate the mapping of the pins of the Arduino UNO with the corresponding Port Pins of the ATmega328P 28-Pin DIP. Describe each pin involved with its function. Explain which facilities/features of these microcontroller pins make the internet connection possible.

Image:

ATmega328P 28-Pin DIP, SPI pins 10/11/12/13 annotated

Answer:

The Ethernet Shield stacks directly onto the Arduino UNO’s headers and communicates with its on-board W5100 Ethernet controller chip using the ATmega328P’s hardware SPI (Serial Peripheral Interface) bus, which occupies four fixed pins: Digital Pin 10 (SS – Slave Select, PB2, physical DIP pin 16) selects/enables the W5100 chip for communication; Digital Pin 11 (MOSI – Master Out Slave In, PB3, DIP pin 17) carries data from the Arduino to the W5100; Digital Pin 12 (MISO – Master In Slave Out, PB4, DIP pin 18) carries data from the W5100 back to the Arduino; and Digital Pin 13 (SCK – Serial Clock, PB5, DIP pin 19) provides the clock signal that synchronises the transfer. These four pins are reserved by the shield and cannot be used for LEDs or other digital I/O while it is mounted. What makes this possible is the ATmega328P’s built-in hardware SPI peripheral, which shifts data in and out synchronously with the clock signal at high speed without needing the CPU to bit-bang each bit — exactly the interface the W5100 chip expects, letting the Arduino exchange Ethernet packet data with it many times per second.

Question 18.d: Explain the following functions in detail.

(18.d.1) Serial.begin(9600);

(18.d.2) Ethernet.begin(mac, ip);

(18.d.3) Ethernet.hardwareStatus()

(18.d.4) Ethernet.linkStatus()

(18.d.5) server.begin();

(18.d.6) EthernetServer server(80);

(18.d.7) Serial.println();

(18.d.8) Serial.print();

(18.d.9) client.println();

(18.d.10) client.print();

Answer:

(18.d.1) Opens the USB serial port and sets its communication speed to 9600 baud, so the board can send text to, and receive text from, the IDE’s Serial Monitor.

(18.d.2) Initialises the Ethernet library and configures the shield’s network interface with the given MAC address and static IP address, bringing the network connection up.

(18.d.3) Returns a value identifying which Ethernet controller chip (e.g. W5100) was detected on the shield, or reports that no shield hardware was found; used here to confirm the shield is recognised.

(18.d.4) Reports the physical-layer link state of the Ethernet cable (LinkON, LinkOFF, or Unknown), i.e. whether a cable is properly plugged in and a link has been established with the router/switch.

(18.d.5) Starts the EthernetServer object listening for incoming client (browser) TCP connections on its configured port.

(18.d.6) Declares/constructs a server object bound to TCP port 80, the standard port for HTTP web traffic.

(18.d.7) Writes the given text to the USB serial connection followed by a newline, for viewing in the Serial Monitor.

(18.d.8) Writes the given text to the USB serial connection without adding a newline afterward.

(18.d.9) Writes the given text, followed by a newline, out over the TCP connection to the connected web browser (building up the HTTP response).

(18.d.10) Writes the given text over the TCP connection to the connected browser without adding a newline, used to build up the HTTP response body.

Experiment 19: Web Page Shows Only Two Labelled Sensors

Aim / Task

Begin with the program of Experiment 18. Modify it so that the web page displays only two analog inputs, each with a descriptive label: (a) connect the LDR divider to A1 and display its value with the text “The current LDR value is …”; (b) connect the potentiometer to A4 and display its value with the text “The current Potentiometer value is …”. Vary both inputs (light on the LDR, position of the potentiometer) and observe the change reflected at the board’s web address.

Circuit Diagram / Photograph

Program Code — Experiment_19

#include <SPI.h> #include <Ethernet.h> byte mac[] = { 0xDE, 0xAD, 0xBE, 0xEF, 0xFE, 0xED }; IPAddress ip(192, 168, 1, 177); EthernetServer server(80); void setup() { Serial.begin(9600); Ethernet.begin(mac, ip); Serial.print("Hardware status: "); Serial.println(Ethernet.hardwareStatus()); Serial.print("Link status: "); Serial.println(Ethernet.linkStatus()); server.begin(); Serial.print("Server is at "); Serial.println(Ethernet.localIP()); } void loop() { EthernetClient client = server.available(); if (client) { boolean blankLine = true; while (client.connected()) { if (client.available()) { char c = client.read(); if (c == '\n' && blankLine) { client.println("HTTP/1.1 200 OK"); client.println("Content-Type: text/html"); client.println("Connection: close"); client.println(); client.println("<!DOCTYPE HTML><html>"); client.print("The current LDR value is "); client.print(analogRead(A1)); client.println("<br />"); client.print("The current Potentiometer value is "); client.print(analogRead(A4)); client.println("<br />"); client.println("</html>"); break; } blankLine = (c == '\n') ? true : (c == '\r') ? blankLine : false; } } delay(1); client.stop(); } }

Questions

Question 19.a: Prepare a video recording with appropriate voice over explaining the execution of this program. Save the video in any folder/drive and include the link to that video in this report.

Video Link: ____________________________________________________________

Answer:

The video shows the same web page refreshed after changing the light falling on the LDR and after turning the potentiometer knob: the line “The current LDR value is …” updates to reflect the new light level, and “The current Potentiometer value is …” updates to reflect the new knob position, confirming both labelled analog readings are live.

Question 19.b: Include your new c program in your report.

#include <SPI.h> #include <Ethernet.h> byte mac[] = { 0xDE, 0xAD, 0xBE, 0xEF, 0xFE, 0xED }; IPAddress ip(192, 168, 1, 177); EthernetServer server(80); void setup() { Serial.begin(9600); Ethernet.begin(mac, ip); Serial.print("Hardware status: "); Serial.println(Ethernet.hardwareStatus()); Serial.print("Link status: "); Serial.println(Ethernet.linkStatus()); server.begin(); Serial.print("Server is at "); Serial.println(Ethernet.localIP()); } void loop() { EthernetClient client = server.available(); if (client) { boolean blankLine = true; while (client.connected()) { if (client.available()) { char c = client.read(); if (c == '\n' && blankLine) { client.println("HTTP/1.1 200 OK"); client.println("Content-Type: text/html"); client.println("Connection: close"); client.println(); client.println("<!DOCTYPE HTML><html>"); client.print("The current LDR value is "); client.print(analogRead(A1)); client.println("<br />"); client.print("The current Potentiometer value is "); client.print(analogRead(A4)); client.println("<br />"); client.println("</html>"); break; } blankLine = (c == '\n') ? true : (c == '\r') ? blankLine : false; } } delay(1); client.stop(); } }

Answer:

See the program code included above.

Question 19.c: Describe the changes you have made in the program.

Answer:

Starting from the Experiment 18 sketch, the for loop that printed all six analog inputs (analog input 0 … analog input 5) was replaced with two explicit, labelled reads: analogRead(A1) (the LDR divider), printed with the text “The current LDR value is …”, and analogRead(A4) (the potentiometer wiper), printed with the text “The current Potentiometer value is …”. No other part of the sketch — the MAC/IP setup, the server object, or the HTTP request-parsing loop — was changed.

Experiment 20: Potentiometer to Serial Monitor and ThingSpeak

Aim / Task

Begin with the program of Experiment 19. Modify it so that the potentiometer, connected to A0, has its value displayed on the Arduino IDE’s Serial Monitor as well as uploaded to the website ThingSpeak.com.

Circuit Diagram / Photograph

Program Code — Experiment_20

#include <SPI.h> #include <Ethernet.h> byte mac[] = { 0xDE, 0xAD, 0xBE, 0xEF, 0xFE, 0xED }; EthernetClient client; char server[] = "api.thingspeak.com"; String writeAPIKey = "YOUR_THINGSPEAK_WRITE_API_KEY"; // from your channel's API Keys tab unsigned long lastUpdate = 0; void setup() { Serial.begin(9600); Ethernet.begin(mac); // DHCP; use Ethernet.begin(mac, ip) if your router has none } void loop() { int potValue = analogRead(A0); Serial.print("Potentiometer value: "); Serial.println(potValue); if (millis() - lastUpdate > 15000) { // ThingSpeak free tier: 1 update / 15 s minimum lastUpdate = millis(); if (client.connect(server, 80)) { String body = "api_key=" + writeAPIKey + "&field1=" + String(potValue); client.println("POST /update HTTP/1.1"); client.println("Host: api.thingspeak.com"); client.println("Connection: close"); client.println("Content-Type: application/x-www-form-urlencoded"); client.print("Content-Length: "); client.println(body.length()); client.println(); client.println(body); client.stop(); } } delay(1000); }

Questions

Question 20.a: Prepare a video recording with appropriate voice over explaining the execution of this program. Save the video in any folder/drive and include the link to that video in this report.

Video Link: ____________________________________________________________

Answer:

The video shows the potentiometer knob being turned while two things are observed simultaneously: the Serial Monitor (opened via Tools → Serial Monitor at 9600 baud) prints a new “Potentiometer value: …” line roughly once a second, and the ThingSpeak channel’s Field 1 chart (refreshed in a browser) shows a new point appear roughly every 15 seconds, tracking the same knob movements with a short delay.

Question 20.b: Include your new c program in your report.

#include <SPI.h> #include <Ethernet.h> byte mac[] = { 0xDE, 0xAD, 0xBE, 0xEF, 0xFE, 0xED }; EthernetClient client; char server[] = "api.thingspeak.com"; String writeAPIKey = "YOUR_THINGSPEAK_WRITE_API_KEY"; // from your channel's API Keys tab unsigned long lastUpdate = 0; void setup() { Serial.begin(9600); Ethernet.begin(mac); // DHCP; use Ethernet.begin(mac, ip) if your router has none } void loop() { int potValue = analogRead(A0); Serial.print("Potentiometer value: "); Serial.println(potValue); if (millis() - lastUpdate > 15000) { // ThingSpeak free tier: 1 update / 15 s minimum lastUpdate = millis(); if (client.connect(server, 80)) { String body = "api_key=" + writeAPIKey + "&field1=" + String(potValue); client.println("POST /update HTTP/1.1"); client.println("Host: api.thingspeak.com"); client.println("Connection: close"); client.println("Content-Type: application/x-www-form-urlencoded"); client.print("Content-Length: "); client.println(body.length()); client.println(); client.println(body); client.stop(); } } delay(1000); }

Answer:

See the program code included above.

Question 20.c: Describe the changes you have made in the program. Write down the differences in the programs for Experiments 19 and 20.

Answer:

Compared with Experiment 19, the local web-server code (EthernetServer, the client.available()/client.read() request-parsing loop, and the HTML page output) was removed entirely, since this experiment no longer serves a local web page. In its place, an EthernetClient object opens an outbound TCP connection to api.thingspeak.com on port 80 and issues an HTTP POST to /update with the potentiometer’s reading in the request body, gated by an if (millis() - lastUpdate > 15000) check so uploads respect ThingSpeak’s free-tier minimum update interval of 15 seconds. A Serial.println() of the same reading was also added so the value can be watched locally on the Serial Monitor at the same time it is being uploaded.

In short: Experiment 19 only serves data locally over the LAN, on request, from two sensors (A1 and A4); Experiment 20 reads only one sensor (A0) but actively pushes its value out to the internet (and to the Serial Monitor) at regular intervals, instead of waiting for a browser to ask for it.

Question 20.d: Write the steps for how you will open an account on ThingSpeak.com.

Answer:

1) Go to thingspeak.com and click Sign Up (or Get Started For Free). 2) Enter an email address, name and password (or sign in with an existing MathWorks account) and verify the email address. 3) Once logged in, click New Channel from the Channels → My Channels page. 4) Give the channel a name, enable Field 1 (and any other fields needed) with a label such as “Potentiometer”, and click Save Channel. 5) Open the new channel and go to its API Keys tab. 6) Copy the Write API Key shown there — this is the key the Arduino sketch uses (as writeAPIKey) to authenticate its POST requests to this channel.

Question 20.e: Describe the details in steps about how you will send the data from the A0 pin of the Arduino UNO to this website.

Answer:

Each pass through loop(), the sketch reads the potentiometer with potValue = analogRead(A0);. If more than 15000 ms have passed since the last upload, it opens a TCP connection to api.thingspeak.com on port 80 with client.connect(server, 80). It then builds the request body, body = "api_key=" + writeAPIKey + "&field1=" + String(potValue);, and sends an HTTP POST request: a request line (POST /update HTTP/1.1), Host/Connection/Content-Type headers, a Content-Length header giving the body’s length, a blank line, and finally the body itself — all written out with client.println()/client.print(). ThingSpeak’s server receives this POST, reads the api_key to identify the channel and the field1 value to log, and stores it as a new datapoint on that channel’s Field 1 chart, timestamped at the moment it was received.

Question 20.f: Describe the details of how you are sending the data to the Serial Monitor.

Answer:

Independently of the ThingSpeak upload, every iteration of loop() calls Serial.println(potValue); immediately after reading the potentiometer. This writes the current ADC reading (0–1023), followed by a newline, out over the Arduino’s USB serial connection at the 9600 baud rate set by Serial.begin(9600) in setup(). Opening Tools → Serial Monitor in the Arduino IDE (also set to 9600 baud) displays each new value as it is printed, roughly once per second (per the delay(1000); at the end of loop()), giving an immediate local readout of the sensor alongside the periodic ThingSpeak upload.