# Experiment Set 1 — Step-by-Step Lab Guide (Arduino UNO)

Submission due: 21/08/2026. Every experiment lists wiring as a flow of single steps,

each step connecting exactly one input entity to one output entity (From → To). Follow

the steps in order — the "To" of one step is usually the "From" of the next step in the same

chain. Nothing is skipped or referred back to "same as before" — each experiment can be built

from a completely empty breadboard using only its own table.

This guide tells you how to build and run each experiment. The graded report

questions from the PDF are listed at the end of each experiment as a checklist — answer

those yourself (they need your own explanation, video, and calculations).

# 1. Components Used (from images/)

| Component | Photo | Pins / Legs |
| --- | --- | --- |
| Arduino UNO | images/arduino-uno.jpeg | Left header: 5V, RESET, 3.3V, 5V, GND, GND, VIN. Bottom header: A0–A5. Right headers: SCL, SDA, AREF, GND, 13, 12, 11~, 10~, 9~, 8 and 7, 6~, 5~, 4, 3~, 2, TX→1, RX←0. ~ = PWM-capable pin. Onboard LED labelled "L" sits next to pin 13. |
| Breadboard | images/breadboard.jpeg | Top/bottom rails: red `+`, blue `−` (full length). Center terminal strips: rows A–E and F–J, columns 1–60. Every column A–E is one electrical node (same for F–J); A–E is NOT connected to F–J (center gap). |
| LED | images/led-light.jpeg | Long leg = anode (+), short leg = cathode (−). |
| Resistor | images/resistor.jpeg | 2 legs, no polarity. |
| Push button switch | images/switch.jpeg | 4 legs: the 2 legs on the same side of the switch body are always internally connected to each other; the 2 sides connect to each other only while pressed. |
| Jumper wires | images/wire.jpeg | Male–male wires for every connection below. |
| USB cable | images/arduino-connector.jpeg | USB-B end → Arduino, USB-A end → PC. |
| Potentiometer | images/potentiometer.jpeg | 3 legs (2 outer + 1 wiper). Not used in Experiments 1–10. |
| LDR module | images/light-intensity-control-senor.jpeg | Labelled DO, GND, VCC. Not used in Experiments 1–10. |
| Bench power supply | images/power-supply.jpeg | Not needed — UNO is powered by USB. |

# 2. One-Time Software Setup

| Step | From | To |
| --- | --- | --- |
| 1 | Arduino UNO USB port | USB cable (USB-B end) |
| 2 | USB cable (USB-A end) | PC USB port |
| 3 | IDE menu Tools → Board | Select Arduino AVR Boards → Arduino Uno |
| 4 | IDE menu Tools → Port | Select COM3 (Arduino UNO) *(use whichever COM number appears)* |

(Install the Arduino IDE first, from https://www.arduino.cc/, if not already installed.)

# 3. Breadboard Layout Key (used by every experiment below)

Component legs (LED legs, resistor legs) always go into row D (upper block) or row G (lower block).

Jumper wires always start from row B or row I, in the *same column* as the component leg they connect to (row B and row D in the same column are electrically the same node).

Power rails: red strip = +, blue strip = −.

# 4. Experiment-by-Experiment Wiring

## Experiment 1 — Blink (Pin 13)

| Step | From | To |
| --- | --- | --- |
| 1 | Arduino UNO USB port | PC USB port (power on the board) |
| 2 | LED long leg (anode) | Breadboard row D, column 5 |
| 3 | LED short leg (cathode) | Breadboard row D, column 6 |
| 4 | Resistor leg 1 | Breadboard row C, column 5 |
| 5 | Resistor leg 2 | Breadboard row C, column 9 |
| 6 | Breadboard row B, column 9 | Arduino Digital Pin 13 |
| 7 | Breadboard row B, column 6 | Breadboard blue (−) rail |
| 8 | Breadboard blue (−) rail | Arduino GND pin |

Note: column 5 holds both the LED anode and resistor leg 1 — they share one electrical node. Also watch the onboard LED labelled "L" next to pin 13 on the Arduino board — it blinks in sync with the external LED, no separate wiring needed for it.

Resistor value (Q1.c): R_min = (5V − 2V) / 0.02A ≈ 150 Ω → use 220 Ω or 330 Ω.

Code — sketch name `Experiment_1`:

void setup() {
  pinMode(13, OUTPUT);
}
void loop() {
  digitalWrite(13, HIGH);
  delay(1000);
  digitalWrite(13, LOW);
  delay(1000);
}

Report checklist: 1.a (video), 1.b (setup/loop/pinMode/digitalWrite/delay), 1.c (resistor calc, done above), 1.d–1.o (UNO datasheet lookups + Mega/Nano comparison).

## Experiment 2 — 40% Duty Cycle (Pin 13)

| Step | From | To |
| --- | --- | --- |
| 1 | Arduino UNO USB port | PC USB port |
| 2 | LED long leg (anode) | Breadboard row D, column 5 |
| 3 | LED short leg (cathode) | Breadboard row D, column 6 |
| 4 | Resistor leg 1 | Breadboard row C, column 5 |
| 5 | Resistor leg 2 | Breadboard row C, column 9 |
| 6 | Breadboard row B, column 9 | Arduino Digital Pin 13 |
| 7 | Breadboard row B, column 6 | Breadboard blue (−) rail |
| 8 | Breadboard blue (−) rail | Arduino GND pin |

(Hardware is identical to Experiment 1 — only the code timing changes below.)

Code:

void setup() {
  pinMode(13, OUTPUT);
}
void loop() {
  digitalWrite(13, HIGH);
  delay(400);   // ON 40% of 1000 ms
  digitalWrite(13, LOW);
  delay(600);   // OFF 60% of 1000 ms
}

Report checklist: 2.a (video), 2.b (paste code), 2.c (describe delay change), 2.d (400/1000 = 40% calc).

## Experiment 3 — Two LEDs, Complementary Duty Cycles (Pin 13, code unchanged from Exp 2)

| Step | From | To |
| --- | --- | --- |
| 1 | Arduino UNO USB port | PC USB port |
| 2 | LED 1 long leg (anode) | Breadboard row D, column 5 |
| 3 | LED 1 short leg (cathode) | Breadboard row D, column 6 |
| 4 | Resistor 1 leg 1 | Breadboard row C, column 5 |
| 5 | Resistor 1 leg 2 | Breadboard row C, column 9 |
| 6 | Breadboard row B, column 9 | Arduino Digital Pin 13 |
| 7 | Breadboard row B, column 6 | Breadboard blue (−) rail |
| 8 | LED 2 short leg (cathode) | Breadboard row D, column 15 |
| 9 | LED 2 long leg (anode) | Breadboard row D, column 16 |
| 10 | Resistor 2 leg 1 | Breadboard row C, column 16 |
| 11 | Resistor 2 leg 2 | Breadboard row C, column 20 |
| 12 | Breadboard row B, column 20 | Breadboard red (+) rail |
| 13 | Breadboard row B, column 15 | Breadboard row B, column 9 |
| 14 | Breadboard blue (−) rail | Arduino GND pin |
| 15 | Breadboard red (+) rail | Arduino 5V pin |

Note: step 13 ties LED 2's cathode node to LED 1's Pin-13 node, so both circuits are driven by the single wire from Pin 13 to column 9 (step 6) — no second wire is plugged into the Arduino pin itself.

Circuit behavior: LED 1 conducts (Pin 13 → 5V path) when Pin 13 = HIGH → ON 40% / OFF 60%. LED 2 conducts (5V rail → Pin 13 sinking) only when Pin 13 = LOW → ON 60% / OFF 40%.

Code: identical to Experiment 2 (re-upload it, or leave it running — no code changes).

Report checklist: 3.a (video), 3.b (explain LED 2 was flipped and tied to the 5V rail so it lights on the LOW phase instead of the HIGH phase).

## Experiment 4 — Fade (Pin 9, PWM)

| Step | From | To |
| --- | --- | --- |
| 1 | Arduino UNO USB port | PC USB port |
| 2 | LED long leg (anode) | Breadboard row D, column 5 |
| 3 | LED short leg (cathode) | Breadboard row D, column 6 |
| 4 | Resistor leg 1 | Breadboard row C, column 5 |
| 5 | Resistor leg 2 | Breadboard row C, column 9 |
| 6 | Breadboard row B, column 9 | Arduino Digital Pin 9 *(marked 9~ on the board)* |
| 7 | Breadboard row B, column 6 | Breadboard blue (−) rail |
| 8 | Breadboard blue (−) rail | Arduino GND pin |

Code — sketch name `Experiment_4`:

int led = 9;
int brightness = 0;
int fadeAmount = 5;
 
void setup() {
  pinMode(led, OUTPUT);
}
void loop() {
  analogWrite(led, brightness);
  brightness = brightness + fadeAmount;
  if (brightness <= 0 || brightness >= 255) {
    fadeAmount = -fadeAmount;
  }
  delay(30);
}

Report checklist: 4.a (video), 4.b (analogWrite), 4.c (explain the boundary-reversal if), 4.d (PWM pins are 3, 5, 6, 9, 10, 11 — the ~ marked pins, tied to hardware timers).

## Experiment 5 — Blink (Pin 13) + Fade (Pin 9) Simultaneously

| Step | From | To |
| --- | --- | --- |
| 1 | Arduino UNO USB port | PC USB port |
| 2 | LED A long leg (anode) | Breadboard row D, column 5 |
| 3 | LED A short leg (cathode) | Breadboard row D, column 6 |
| 4 | Resistor A leg 1 | Breadboard row C, column 5 |
| 5 | Resistor A leg 2 | Breadboard row C, column 9 |
| 6 | Breadboard row B, column 9 | Arduino Digital Pin 13 |
| 7 | Breadboard row B, column 6 | Breadboard blue (−) rail |
| 8 | LED B long leg (anode) | Breadboard row D, column 15 |
| 9 | LED B short leg (cathode) | Breadboard row D, column 16 |
| 10 | Resistor B leg 1 | Breadboard row C, column 15 |
| 11 | Resistor B leg 2 | Breadboard row C, column 19 |
| 12 | Breadboard row B, column 19 | Arduino Digital Pin 9 |
| 13 | Breadboard row B, column 16 | Breadboard blue (−) rail |
| 14 | Breadboard blue (−) rail | Arduino GND pin |

Code:

int led9 = 9, brightness = 0, fadeAmount = 5;
int led13 = 13;
unsigned long previousMillis = 0;
bool led13State = LOW;
 
void setup() {
  pinMode(led9, OUTPUT);
  pinMode(led13, OUTPUT);
}
void loop() {
  analogWrite(led9, brightness);
  brightness += fadeAmount;
  if (brightness <= 0 || brightness >= 255) fadeAmount = -fadeAmount;
 
  if (millis() - previousMillis >= 1000) {
    previousMillis = millis();
    led13State = !led13State;
    digitalWrite(led13, led13State);
  }
  delay(30);
}

Report checklist: 5.a (video), 5.b (paste code), 5.c (added LED B branch on pin 9; pin 13 timing moved from delay() to millis() so it doesn't block the fade).

## Experiment 6 — Blink 4 LEDs Together (Pins 13, 11, 9, 7)

| Step | From | To |
| --- | --- | --- |
| 1 | Arduino UNO USB port | PC USB port |
| 2 | LED 1 long leg (anode) | Breadboard row D, column 5 |
| 3 | LED 1 short leg (cathode) | Breadboard row D, column 6 |
| 4 | Resistor 1 leg 1 | Breadboard row C, column 5 |
| 5 | Resistor 1 leg 2 | Breadboard row C, column 9 |
| 6 | Breadboard row B, column 9 | Arduino Digital Pin 13 |
| 7 | Breadboard row B, column 6 | Breadboard blue (−) rail |
| 8 | LED 2 long leg (anode) | Breadboard row D, column 15 |
| 9 | LED 2 short leg (cathode) | Breadboard row D, column 16 |
| 10 | Resistor 2 leg 1 | Breadboard row C, column 15 |
| 11 | Resistor 2 leg 2 | Breadboard row C, column 19 |
| 12 | Breadboard row B, column 19 | Arduino Digital Pin 11 |
| 13 | Breadboard row B, column 16 | Breadboard blue (−) rail |
| 14 | LED 3 long leg (anode) | Breadboard row D, column 25 |
| 15 | LED 3 short leg (cathode) | Breadboard row D, column 26 |
| 16 | Resistor 3 leg 1 | Breadboard row C, column 25 |
| 17 | Resistor 3 leg 2 | Breadboard row C, column 29 |
| 18 | Breadboard row B, column 29 | Arduino Digital Pin 9 |
| 19 | Breadboard row B, column 26 | Breadboard blue (−) rail |
| 20 | LED 4 long leg (anode) | Breadboard row D, column 35 |
| 21 | LED 4 short leg (cathode) | Breadboard row D, column 36 |
| 22 | Resistor 4 leg 1 | Breadboard row C, column 35 |
| 23 | Resistor 4 leg 2 | Breadboard row C, column 39 |
| 24 | Breadboard row B, column 39 | Arduino Digital Pin 7 |
| 25 | Breadboard row B, column 36 | Breadboard blue (−) rail |
| 26 | Breadboard blue (−) rail | Arduino GND pin |

Code:

int pins[] = {13, 11, 9, 7};
 
void setup() {
  for (int i = 0; i < 4; i++) pinMode(pins[i], OUTPUT);
}
void loop() {
  for (int i = 0; i < 4; i++) digitalWrite(pins[i], HIGH);
  delay(1000);
  for (int i = 0; i < 4; i++) digitalWrite(pins[i], LOW);
  delay(1000);
}

Report checklist: 6.a (video), 6.b (paste code), 6.c (describe 4-LED wiring and array-based code).

## Experiment 7 — Sequential ON, Then All OFF (Pins 13, 11, 9, 7)

| Step | From | To |
| --- | --- | --- |
| 1 | Arduino UNO USB port | PC USB port |
| 2 | LED 1 long leg (anode) | Breadboard row D, column 5 |
| 3 | LED 1 short leg (cathode) | Breadboard row D, column 6 |
| 4 | Resistor 1 leg 1 | Breadboard row C, column 5 |
| 5 | Resistor 1 leg 2 | Breadboard row C, column 9 |
| 6 | Breadboard row B, column 9 | Arduino Digital Pin 13 |
| 7 | Breadboard row B, column 6 | Breadboard blue (−) rail |
| 8 | LED 2 long leg (anode) | Breadboard row D, column 15 |
| 9 | LED 2 short leg (cathode) | Breadboard row D, column 16 |
| 10 | Resistor 2 leg 1 | Breadboard row C, column 15 |
| 11 | Resistor 2 leg 2 | Breadboard row C, column 19 |
| 12 | Breadboard row B, column 19 | Arduino Digital Pin 11 |
| 13 | Breadboard row B, column 16 | Breadboard blue (−) rail |
| 14 | LED 3 long leg (anode) | Breadboard row D, column 25 |
| 15 | LED 3 short leg (cathode) | Breadboard row D, column 26 |
| 16 | Resistor 3 leg 1 | Breadboard row C, column 25 |
| 17 | Resistor 3 leg 2 | Breadboard row C, column 29 |
| 18 | Breadboard row B, column 29 | Arduino Digital Pin 9 |
| 19 | Breadboard row B, column 26 | Breadboard blue (−) rail |
| 20 | LED 4 long leg (anode) | Breadboard row D, column 35 |
| 21 | LED 4 short leg (cathode) | Breadboard row D, column 36 |
| 22 | Resistor 4 leg 1 | Breadboard row C, column 35 |
| 23 | Resistor 4 leg 2 | Breadboard row C, column 39 |
| 24 | Breadboard row B, column 39 | Arduino Digital Pin 7 |
| 25 | Breadboard row B, column 36 | Breadboard blue (−) rail |
| 26 | Breadboard blue (−) rail | Arduino GND pin |

(Hardware identical to Experiment 6 — only the code sequencing changes below.)

Code:

int pins[] = {13, 11, 9, 7};
 
void setup() {
  for (int i = 0; i < 4; i++) pinMode(pins[i], OUTPUT);
}
void loop() {
  for (int i = 0; i < 4; i++) {
    digitalWrite(pins[i], HIGH);
    delay(2000);
  }
  for (int i = 0; i < 4; i++) digitalWrite(pins[i], LOW);
  delay(2000);
}

Report checklist: 7.a (video), 7.b (paste code), 7.c (describe: same wiring as Exp 6, code now staggers the ON sequence 2 s apart, then clears all four together).

## Experiment 8 — Rotating Single LED (Pins 13, 11, 9, 7)

| Step | From | To |
| --- | --- | --- |
| 1 | Arduino UNO USB port | PC USB port |
| 2 | LED 1 long leg (anode) | Breadboard row D, column 5 |
| 3 | LED 1 short leg (cathode) | Breadboard row D, column 6 |
| 4 | Resistor 1 leg 1 | Breadboard row C, column 5 |
| 5 | Resistor 1 leg 2 | Breadboard row C, column 9 |
| 6 | Breadboard row B, column 9 | Arduino Digital Pin 13 |
| 7 | Breadboard row B, column 6 | Breadboard blue (−) rail |
| 8 | LED 2 long leg (anode) | Breadboard row D, column 15 |
| 9 | LED 2 short leg (cathode) | Breadboard row D, column 16 |
| 10 | Resistor 2 leg 1 | Breadboard row C, column 15 |
| 11 | Resistor 2 leg 2 | Breadboard row C, column 19 |
| 12 | Breadboard row B, column 19 | Arduino Digital Pin 11 |
| 13 | Breadboard row B, column 16 | Breadboard blue (−) rail |
| 14 | LED 3 long leg (anode) | Breadboard row D, column 25 |
| 15 | LED 3 short leg (cathode) | Breadboard row D, column 26 |
| 16 | Resistor 3 leg 1 | Breadboard row C, column 25 |
| 17 | Resistor 3 leg 2 | Breadboard row C, column 29 |
| 18 | Breadboard row B, column 29 | Arduino Digital Pin 9 |
| 19 | Breadboard row B, column 26 | Breadboard blue (−) rail |
| 20 | LED 4 long leg (anode) | Breadboard row D, column 35 |
| 21 | LED 4 short leg (cathode) | Breadboard row D, column 36 |
| 22 | Resistor 4 leg 1 | Breadboard row C, column 35 |
| 23 | Resistor 4 leg 2 | Breadboard row C, column 39 |
| 24 | Breadboard row B, column 39 | Arduino Digital Pin 7 |
| 25 | Breadboard row B, column 36 | Breadboard blue (−) rail |
| 26 | Breadboard blue (−) rail | Arduino GND pin |

(Hardware identical to Experiments 6 & 7 — only the code changes below.)

Code:

int pins[] = {13, 11, 9, 7};
 
void setup() {
  for (int i = 0; i < 4; i++) pinMode(pins[i], OUTPUT);
}
void loop() {
  for (int i = 0; i < 4; i++) {
    digitalWrite(pins[i], HIGH);
    delay(2000);
    digitalWrite(pins[i], LOW);
  }
}

Report checklist: 8.a (video), 8.b (paste code), 8.c (describe changes vs Exp 7), 8.d–8.f (LED datasheet: Green material combo, Red peak forward current, Blue peak wavelength), 8.g (why different materials give different LED colors — bandgap energy vs photon wavelength, with diagram), 8.h (define Luminosity + SI unit: lumen).

## Experiment 9 — Push Button Controls LED (Pin 13 LED, Pin 2 button)

| Step | From | To |
| --- | --- | --- |
| 1 | Arduino UNO USB port | PC USB port |
| 2 | LED long leg (anode) | Breadboard row D, column 5 |
| 3 | LED short leg (cathode) | Breadboard row D, column 6 |
| 4 | Resistor leg 1 | Breadboard row C, column 5 |
| 5 | Resistor leg 2 | Breadboard row C, column 9 |
| 6 | Breadboard row B, column 9 | Arduino Digital Pin 13 |
| 7 | Breadboard row B, column 6 | Breadboard blue (−) rail |
| 8 | Switch leg (top-left) | Breadboard row D, column 45 |
| 9 | Switch leg (top-right) | Breadboard row D, column 47 |
| 10 | Switch leg (bottom-left) | Breadboard row G, column 45 |
| 11 | Switch leg (bottom-right) | Breadboard row G, column 47 |
| 12 | Breadboard row C, column 45 | Breadboard red (+) rail |
| 13 | Breadboard row H, column 45 | Arduino Digital Pin 2 |
| 14 | Pull-down resistor leg 1 | Breadboard row H, column 47 |
| 15 | Pull-down resistor leg 2 | Breadboard row H, column 50 |
| 16 | Breadboard row H, column 50 | Breadboard blue (−) rail |
| 17 | Breadboard blue (−) rail | Arduino GND pin |
| 18 | Breadboard red (+) rail | Arduino 5V pin |

Notes: the switch straddles the breadboard's center gap (legs in row D and row G). Its top-left and top-right legs (step 8, 9) are internally shorted into one "top node"; its bottom-left and bottom-right legs (step 10, 11) are internally shorted into one "bottom node." Pressing the button connects the top node to the bottom node. Confirm this pull-down circuit with your instructor before wiring, per the slide.

Logic: button not pressed → Pin 2 pulled to GND through the resistor → reads LOW. Button pressed → top node (5V) connects to the bottom node → Pin 2 reads HIGH.

Code — sketch name `Experiment_9`:

const int buttonPin = 2;
const int ledPin = 13;
int buttonState = 0;
 
void setup() {
  pinMode(ledPin, OUTPUT);
  pinMode(buttonPin, INPUT);
}
void loop() {
  buttonState = digitalRead(buttonPin);
  if (buttonState == HIGH) {
    digitalWrite(ledPin, HIGH);
  } else {
    digitalWrite(ledPin, LOW);
  }
}

Note: with this exact code and pull-down wiring, the LED is ON while pressed. If the target behavior is "LED OFF while pressed" as worded on the slide, swap the two branches (see Experiment 10) or rewire as a pull-up instead.

Report checklist: 9.a (video), 9.b (digitalRead), 9.c (draw the button circuit; explain how it produces logic HIGH/LOW).

## Experiment 10 — Push Button Turns LED Off, Otherwise ON

| Step | From | To |
| --- | --- | --- |
| 1 | Arduino UNO USB port | PC USB port |
| 2 | LED long leg (anode) | Breadboard row D, column 5 |
| 3 | LED short leg (cathode) | Breadboard row D, column 6 |
| 4 | Resistor leg 1 | Breadboard row C, column 5 |
| 5 | Resistor leg 2 | Breadboard row C, column 9 |
| 6 | Breadboard row B, column 9 | Arduino Digital Pin 13 |
| 7 | Breadboard row B, column 6 | Breadboard blue (−) rail |
| 8 | Switch leg (top-left) | Breadboard row D, column 45 |
| 9 | Switch leg (top-right) | Breadboard row D, column 47 |
| 10 | Switch leg (bottom-left) | Breadboard row G, column 45 |
| 11 | Switch leg (bottom-right) | Breadboard row G, column 47 |
| 12 | Breadboard row C, column 45 | Breadboard red (+) rail |
| 13 | Breadboard row H, column 45 | Arduino Digital Pin 2 |
| 14 | Pull-down resistor leg 1 | Breadboard row H, column 47 |
| 15 | Pull-down resistor leg 2 | Breadboard row H, column 50 |
| 16 | Breadboard row H, column 50 | Breadboard blue (−) rail |
| 17 | Breadboard blue (−) rail | Arduino GND pin |
| 18 | Breadboard red (+) rail | Arduino 5V pin |

(Hardware identical to Experiment 9 — only the code branches are swapped below.)

Code:

const int buttonPin = 2;
const int ledPin = 13;
int buttonState = 0;
 
void setup() {
  pinMode(ledPin, OUTPUT);
  pinMode(buttonPin, INPUT);
}
void loop() {
  buttonState = digitalRead(buttonPin);
  if (buttonState == HIGH) {
    digitalWrite(ledPin, LOW);   // pressed -> LED off
  } else {
    digitalWrite(ledPin, HIGH);  // not pressed -> LED on
  }
}

Report checklist: 10.a (video), 10.b (paste code), 10.c (describe: wiring unchanged from Exp 9, only the HIGH/LOW branches were swapped).

# 5. Quick Pin-Use Summary

| Experiment | Digital pin(s) used | Extra hardware beyond LED+resistor |
| --- | --- | --- |
| 1 | 13 | — |
| 2 | 13 | — |
| 3 | 13 (both LEDs, one reversed) | 2nd LED wired reverse-polarity to 5V rail |
| 4 | 9 (PWM) | — |
| 5 | 13 + 9 | — |
| 6 | 13, 11, 9, 7 | — |
| 7 | 13, 11, 9, 7 | — |
| 8 | 13, 11, 9, 7 | — |
| 9 | 13 (LED) + 2 (button) | Push button + pull-down resistor |
| 10 | 13 (LED) + 2 (button) | Same as Experiment 9 |