# Experiment 3 — Two LEDs, Complementary Duty Cycles

## Voice-Over Script

Word count: 73    |    Approx. read time: 30 seconds at a natural pace (~145 wpm)

Experiment three: complementary duty cycles from one pin. The code is untouched. LED one still runs from pin thirteen to ground, lighting on the high phase — forty percent. LED two is reversed: its anode goes through a resistor to five volts, and its cathode ties to the same pin thirteen node. It only conducts when pin thirteen sinks current, that is when the pin is low — so it lights sixty percent.