# Experiment 10 — Push Button Turns LED Off, Otherwise ON

## Voice-Over Script

Word count: 73    |    Approx. read time: 30 seconds at a natural pace (~145 wpm)

Experiment ten: inverting the button logic. Not a single wire changes from experiment nine — same LED on pin thirteen, same button and pull-down resistor on pin two. Only the two branches of the if statement are swapped. Now a HIGH reading, meaning the button is pressed, writes LOW and switches the LED off; releasing it returns pin two to LOW and the LED comes back on. A software-only inversion of the behaviour.